| Client: |  |  | Contract No: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Project: |  |  | Document No: |  |  | Revision: |  |
| Area/System No: |  |  | Area/System Description: |  |  |  |  |

| Reference Document |  |  | Controlling Document |  |  | Verifying Documents |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Title |  | No. | Title |  | No. | Title |
|  | BGER Project Quality Management Plan (PQMP) |  | AS 1554.1 | Structural Steel Welding Part 1 |  |  | Material Certificates |
| AS 4100 | Steel Structure |  | AS 1674 | Safety in welding and allied process |  | BGER-QA-CHL-0001 | Material Inspection Report |
|  | Weld Procedure Specifications (WPS) |  | AS 2207 | Non Detructive Testing – UT of Fusion Welded Joints in Carbon and Low Alloy Steel |  |  | Approved Lift Studies |
|  | Welder Qualification Register (WQR) |  | AS 1657 | Fixed platforms, walkways, stairways and ladder – Design, construction and installation |  |  | Survey Report |
|  |  |  | AS 5131 | Structural Steelwork Fabrication |  |  |  |
|  |  |  |  |  |  |  | NDT Report & Weld Map |
|  |  |  |  | AFC Dawings |  |  |  |
|  |  |  |  | Client’s specifications |  | BGER-QA-CHL-0002 | Coating & Surface Treatment Checklist |

| Legend |  |  | Approval and Change History |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Verification Activity |  |  | Rev | Details | Date | Prepared by (BGER) | Client Approval (XXX) |
| H – Hold Point | Work shall not proceed past the Hold Point until released by the Organisation imposing the Hold Point. 48 hrs. notice required. |  |  |  |  |  |  |
| W – Witness Point | An Inspection Point that may be witnessed by the Organisation imposing the Witness Point. 48 hrs. notice required. |  |  |  |  |  |  |
| I – Inspection | Formal inspection activity to be undertaken and recorded. |  |  |  |  |  |  |
| S – Surveillance | An activity that is subject to on-going monitoring |  |  |  |  |  |  |
| R – Review | Review of text reports/records or other evidence of compliance. |  |  |  |  |  |  |
| Responsible Person (RP) |  |  |  |  |  |  |  |
| SUP | Supervisor |  |  |  |  |  |  |
| QR | Quality Representative (Project Engineer) |  |  |  |  |  |  |
| SUR | Surveyor |  |  |  |  |  |  |
| NDTT | NDT Technician |  |  |  |  |  |  |

| Item | Activity/Operations/Stage No. | Controlling Document/Procedure | Standard, Spec, Drawing Ref or Acceptance Criteria | Verifying Document/Report/ Certificate | Verification Activity by |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | BGER RP | BGER/ sign | Client/ sign |  |
|  | Approvals |  |  |  |  |  |  |  |
|  | Prepare BGER’s ITP to suit and submit to Client for approval | PQMP | ITP | Approved ITP | QR | H | H |  |
|  | Submit and obtain approval for Weld Procedure Specifications and Welder Qualifications | PQMP | AS 1554 | Approved WPS and WQR WPS and WQR Register | QR | H | H |  |
|  | Submit NATA certification/accreditation and NDT procedures. Submit NDT technician qualifications. | PQMP | AS 1554 | Approved NDT Procedures Approved NDT Qualifications | QR | H | H |  |
|  | Material Receipt |  |  |  |  |  |  |  |
|  | Check material for damage due to delivery and fabrication defect | PQMP | Isometric drawings, Delivery Packing List | Material Inspection Report | SUP | I, R | R |  |
|  | Confirm correct material received | PQMP | AFC Drawings and Client Specs | Material Certificates | SUP | I, R | R |  |
|  | Check that correct quantities and dimensions of material and welding consumables are delivered | PQMP | AFC Drawings, Bill of Material | Material Inspection Report, Welding Consumables Material Certificates | SUP | I, R | R |  |
|  | Confirm that the materials are stored in a controlled area. | PQMP | ITP | Material Receipt Checklist | SUP | I, R | S |  |
|  | Materials Processing |  |  |  |  |  |  |  |
|  | Check material type and identification, confirm | PQMP | AFC Drawings, Client specs | Material Inspection Report, Material Certificates | SUP | I | R |  |
|  | Material identified by AFC Drawing and Mark Number | PQMP | AFC drawings | Material Inspection Report | SUP | I | R |  |
|  | Material cut and ends prepared, free of burrs and sharp edges | PQMP | AS 1554 | Material Inspection Report / Checklist | SUP | I | R |  |
|  | Materials issued for fabrication | PQMP | AFC Drawings | Material Inspection Report | SUP | I | R |  |
|  | Fabrication |  |  |  |  |  |  |  |
|  | Approved WPS and Work Packs/Instructions at job front | PQMP | Approved WPS | Approved WPS | SUP | H | R |  |
|  | Check correct material, weld preparations, consumables | WPS | AFC Drawings AS 1554 | Approved WPS Batch certificates | SUP | I | S |  |
|  | Check fit up, configuration, dimensions and orientation. | PQMP | AFC Drawings | Fabrication Checklist | SUP | I | S |  |
|  | Confirm welder qualification and welding parameters | WQR, WPS | AS 1554 | Approved WQR Approved WPS | SUP | R | S |  |
|  | Check weld root and hot pass | WPS | AS 1554 | Fabrication Checklist | SUP | S | S |  |
|  | Check completed weld | WPS | AS 1554 | Fabrication Checklist | SUP | I | S |  |
|  | Check removal of burrs, slag, spatter, scale and flux | WPS | AS 1554 | Fabrication Checklist | SUP | I | S |  |
|  | Check holes are clean cut | PQMP | AS 1554 | Fabrication Checklist | SUP | I | S |  |
|  | Check for mechanical damage | PQMP | AS 1554 | Fabrication Checklist | SUP | I | S |  |
|  | Final dimensional check | PQMP | AFC Drawings | Fabrication Checklist | SUP | H | S |  |
|  | Inspection & Testing |  |  |  |  |  |  |  |
|  | Perform final inspection: - Visual - Dimensional - Weld inspection - Distortion checks - Sharp edges - Bolt holes - Correct mark number / identification | PQMP | AFC Drawing AS 1554 | Reports/Checklists | SUP/QR | W | R/S |  |
|  | All welds 100% visual scanning All welds 20% visual examination | AS 4100 | AS 1554 |  | SUP/QR | S | R/S |  |
|  | RT/UT and MT/PT of 10% of the length of butt welds >1000mm and 10% of all butt welds <1000mm | PQMP | Approved NDT procedures AS 1554 | NDT Request, Weld Maps, NATA reports | NDTT | W | R/S |  |
|  | MT or PT of 10% of the length of fillet welds >1000mm and 10% of all fillet welds <1000mm | PQMP | Approved NDT procedures AS 1554 | NDT Request, Weld Maps, NATA reports | NDTT | W | R/S |  |
|  | RT/UT and MT/PT of 100% of splice welds | PQMP | Approved NDT procedures AS 1554 | NDT Request, Weld Maps, NATA reports | NDTT | W | R/S |  |
|  | 100% UT/MT/PT of Lifting Lugs | PQMP | Approved NDT procedures AS 1554 | NDT Request, Weld Maps, NATA reports | NDTT | W | R/S |  |
|  | 100% MT or PT of all parent material repair sites | PQMP | Approved NDT procedures AS 1554 | NDT Request, Weld Maps, NATA reports | NDTT | W | R/S |  |
|  | Final Inspection |  |  |  |  |  |  |  |
|  | Confirm all inspections and tests are completed with reports available and conforming to Client specifications | PQMP | AFC Drawings AS 1554, AS 4100 | Reports/Checklists | QR | H | R |  |
|  | Inspect all units prior to delivery to surface treatment subcontractor, confirming delivery docket | PQMP | AFC Drawings | Delivery docket | QR | H | W |  |
|  | Issue IRC and prepare for delivery for surface treatment | PQMP | SOW, Client specs | IRC | QR | H | R |  |
|  | Packaging and Delivery |  |  |  |  |  |  |  |
|  | Check all items have identification tags fixed | PQMP | Client packing specification | Delivery Docket | SUP/QR | W | R |  |
|  | Check all items have been packed with sufficient dunnage and support for safe transport and unloading | PQMP | Client packing specification | Delivery Docket | SUP/QR | W | R |  |
|  | Quality Documents |  |  |  |  |  |  |  |
|  | Compilation of MDR | PQMP | Client’ Specs | MDR progressive review and sign off | QR | R | R |  |