| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| GENERAL DATA |  |  |  |
| --- | --- | --- | --- |
| GA Drawing |  | Section Drawing |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| Field Checks |  |  |  |  |
|  | ITR C1.001 for Clearing works has been completed and signed off by Contractor & Engineer? |  |  |  |
|  | ITR C1.002 for Excavation works has been completed and signed off by Contractor and Engineer? |  |  |  |
|  | Confirm final finished levels have been achieved for the site prior to landing concrete blocks (survey report) |  |  |  |
|  | Dimensions of concrete blocks supplied in accordance with the relevant drawings? |  |  |  |
|  | Concrete blocks received free from damage and other embedded items checked and correct (Swift lift points) |  |  |  |
| Hold Point | Have items 1-5 been checked and completed? Approval to proceed with concrete block installation |  |  |  |
|  | Ground surface free from any deleterious materials, rubble, etc that may impact on concrete block final levels? |  |  |  |
|  | String line setup for correct position/alignment of concrete blocks |  |  |  |
|  | Tie-Down for Building Options: - Cast-in - 2-M20 350x100 L Bar 250 embedment as per IFC Drawing for pre-cast concrete blocks, or (If option 1 does not suit) - Drill and Epoxy using powers PF PRO or similar approved M20 8.8 Grade Galvanised Threaded Road 250 embedment |  |  |  |
|  | Tie-Down elements vertical/plumb. projection correct & clean and undamaged |  |  |  |
|  | Install Keeper plates as per IFC Drawing |  |  |  |
|  | Other embedded items checked and correct (Swift lift points) |  |  |  |
|  | Survey of all anchor bolts completed |  |  |  |

| REMARKS / REMEDIAL ACTION REQUIRED |  |  |  |
| --- | --- | --- | --- |
| Item | Description | Rectified Date | Signature |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |