| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

- \

| ITP Ref / No.: |  |
| --- | --- |
| Location / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | All permits in place and services located eg. Area Request, Clearing permit, excavation permit (Permits) |  |  |  |  |  |
|  | Define Lot and set out area including (Survey data report): - Fluoro pink tape for Approval Request Boundary, - Fluoro green tape for Clearing boundary, flora/fauna protection zones marked, heritage boundaries marked, - Set out material stockpile locations including segregation zones |  |  |  |  |  |
|  | Ensure vehicles and machinery to be fitted with GPS and loaded with correct files: - Area request, - Clearing permit, - Work boundary and - Exclusions |  |  |  |  |  |
|  | Pre-clearing checks completed: - Walk boundary with Engineer, - Complete JHA and take 5 and - Locate area for topsoil and clearing stockpiles |  |  |  |  |  |
|  | Additional controls implemented for Riparian (watercourse) zones |  |  |  |  |  |
| Hold Point | Have items 1-5 been checked |  |  |  |  |  |
| Clear and Grub |  |  |  |  |  |  |
|  | Clearing (removal of trees, shrubs and vegetation close to surface) completed. |  |  |  |  |  |
|  | Confirm that marked area only has been cleared and all roots and stumps removed (Survey as-built data report) |  |  |  |  |  |
|  | Check for underground services (if applicable), removal of all organic materials/boulders to depth of 300mm in General Earthworks or 600mm underneath road, building & structures from natural surface, subgrade or foundation (whichever is deepest) |  |  |  |  |  |
|  | Stockpiling: Stockpiled material placed in designated area – within disturbance permit |  |  |  |  |  |
| Topsoil Removal |  |  |  |  |  |  |
|  | Set out Area for topsoil removal (Survey data report) |  |  |  |  |  |
|  | Verify area has been cleared in marked boundaries |  |  |  |  |  |
|  | Set out area for material stockpiling (material must not be mixed with material from clear and grub process) |  |  |  |  |  |
|  | All permits in place eg. Dig permit. |  |  |  |  |  |
|  | Confirm that only Topsoil removed in marked area (Survey as-built data report) |  |  |  |  |  |
| Hold Point | Have items 10-13 been checked |  |  |  |  |  |
| Stripping of approved area |  |  |  |  |  |  |
|  | Stripping of topsoil: - Minimum 200mm topsoil removal (100mm only within pipeline or powerline corridor) or in consultation with RTIO EPCM Engineer |  |  |  |  |  |
|  | Stockpiling: - Stockpile topsoil to 2m to reduce foot print of stockpile, stockpile batter slopes to be 1:4 - Remove topsoil and stockpile separately to subsoil where applicable |  |  |  |  |  |
|  | Survey Topsoil stripped area (Survey as-built data report) |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |