| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| Pour Location |  |
| --- | --- |
| Grade / Strength |  |
| Estimated Volume |  |
| Supplier Receipts (Attach receipts with ITR) |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot: |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  |  |  |  |
|  | Foundation excavation (clean up or compaction) |  |  |  |
|  | Plastic vapor barriers sheets are not damaged and clean. - Where specified, place 0.3mm - Polythene sheet ensuring joints are overlapped min 150mm and sealed with 50mm wide adhesive tape. - Nil punctures or visible tears. |  |  |  |
|  | Blinding: - Minimum 15 to 50mm thick or as directed / shown in AFC drawings |  |  |  |
|  | Formwork - finish, line and rigidity - clean up, coating, insulation and - top of concrete line marked |  |  |  |
|  | Chamfer strips shall be 20x20mm unless shown otherwise on the AFC drawings. Check for verticality and stability of upstanding / columns |  |  |  |
|  | Water stops fixed and secure against movement during placing / vibrating of the concrete |  |  |  |
|  | Joint Sealer, Expansion Joint Filler, Construction joints and Block outs as per SS-C-108 |  |  |  |
|  | Cover to reinforcement achieved and concrete blocks, chairs / distancers installed |  |  |  |
|  | Reinforcement free from scale, loose rust, oil, paint, grease or other deleterious matter |  |  |  |
|  | Reinforcement arrangement – splicing, diameter, spacing and fixing |  |  |  |
|  | Construction Safety Caps on exposed reinforcement and bolts |  |  |  |
|  | Anchor bolt diameter, spacing and projection |  |  |  |
|  | Rigidity of anchor bolts and sleeves |  |  |  |
|  | Cast-ins and embedment |  |  |  |
|  | Location of piping, cable ducts and conduits |  |  |  |
|  | Electrical earthing and lightning protection as per design an all connections shall be checked for suitability |  |  |  |
|  | Weather conditions (acceptable) |  |  |  |
|  | Cylinder and slump testing procedures in place |  |  |  |
|  | Curing (membrane forming or water) |  |  |  |

| EPCM APPROVED BY: |  |  |  |
| --- | --- | --- | --- |
| Signature: |  | Date: |  |
| Name: |  | Title: |  |

| OTHER CHECKS |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Method of Placement of Concrete: |  |  |  |
|  | No of concrete samples for compression testing |  |  |  |
|  | Surface finish of concrete: |  |  |  |
|  | Curing Method to be used |  |  |  |
|  | Protection of concrete post pour required: |  |  |  |
|  | Marked up drawing attached showing pour location |  |  |  |

| POUR STARTED | TIME: | DATE: | POUR INSPECTOR: |
| --- | --- | --- | --- |
| POUR FINISHED | TIME: | DATE: | POUR INSPECTOR: |
| ACTUAL VOLUME PLACED |  | WEATHER CONDITIONS: |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. I hereby confirm all Inspection Test Plan (ITP) Hold and Witness points have been completed by the Contractor & Engineer |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |