| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | Verify underlying lot and obtain required permits C1.009 - Clear Grab Topsoil Removal |  |  |  |  |  |
|  | Set out works (Survey Data Report) |  |  |  |  |  |
|  | Verify Material Tests for bedding material & placement - Material to comply to RVS-0000-SPE-Y-001, Section 9.4, Table 6,7 & 8 - Testing Frequency VS-0000-SPE-Y-00, Section 13.2, Table 12 |  |  |  |  |  |
|  | Verify that Method Spec for material and placement approved |  |  |  |  |  |
|  | Verify approval of Rock Protection Material as per RVS-0000-SPE-Y-001, Section, 9.8, Table 11 |  |  |  |  |  |
|  | Verify approval of Geotextile Filter Fabric as per RVS-0000-SPE-Y-001 9.8 Table 14.3.1 - Geotextile fabric to be non-woven, continuous filament, spun bounded, needle punched polyester geotextile compliant to AS 3795. |  |  |  |  |  |
| Hold Point | Have Items 1- 6 been checked |  |  |  |  |  |
|  | Trench alignment pegged |  |  |  |  |  |
|  | Excavation: - Excavation benching below 1.5m cut - Set out Centreline - Dewater when required |  |  |  |  |  |
|  | Prepare bedding for culvert and end treatments RVS-0000-SPE-Y-001, Section14.2.3 |  |  |  |  |  |
|  | Place Culverts RVS-0000-SPE-Y-001, Section14.2.4 |  |  |  |  |  |
|  | Joints RVS-0000-SPE-Y-001, Section14.2.5 and as per manufacturers requirements |  |  |  |  |  |
|  | Verify CSP PVC Geomembrane Wrapping RVS-0000-SPE-Y-001, Section14.2.6: - Minimum of 500mm adhesive-fixed, overlap over the full length of pipe. Overlap to be on top of pipe for inspection. |  |  |  |  |  |
|  | Survey & Engineer Check prior to backfill RVS-0000-SPE-Y-001, Section14.2.8: - Horizontal tolerance ±150mm - Rate of horizontal misalignment must not exceed 10mm in 1m length - Vertical tolerance ±25mm - Culverts are secured to prevent floating |  |  |  |  |  |
| Hold Point | Have Items 1- 6 been checked |  |  |  |  |  |
|  | Culvert Backfill as per RVS-0000-SPE-Y-001, Section14.2.7 Cement Stabilised: - 300mm cover of Cement Stabilised |  |  |  |  |  |
|  | Culvert Backfill Granular Fill, mix design as: - 5% Portland Cement by weight - 83% Fine aggregates and - 10% Water |  |  |  |  |  |
| - 1 | Culvert Backfill General Fill: - Place general fill at 500mm and compact to 300mm layers. - Use equipment that will not damage culvert. - 3no. tests per lift: 93% MMDD & ±3% OMC |  |  |  |  |  |
|  | Box out for rip rap as per RVS-0000-SPE-Y-001, Section14.2.9: - Thickness of rip rap to be as per design drawings and culvert schedule; |  |  |  |  |  |
|  | Rock and Scour, Protection Geotextile, Filter Fabric Placement: - Trim to design levels and smooth and compact underlying material. - Minimum of 1m overlap at joints and secured in place to prevent movement. - Rock must be placed within 4 days of Geo-fab placement |  |  |  |  |  |
|  | Rock and Scour Protection Placement: - Rock and Scour protection as per RVS-0000-SPE-Y-001 table 11 - All rock placed with excavator that it rests firmly upon and does not displace underlying material |  |  |  |  |  |
|  | Work completed: - As built (Survey Data Report) |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |