| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | Obtain excavation & penetration permit |  |  |  |  |  |
|  | Set out the works to the lines and levels shown on the drawings or specified from Client supplied primary survey control points, by licensed surveyor |  |  |  |  |  |
|  | Identify the location of all underground services and structures in the vicinity of the work |  |  |  |  |  |
|  | Work area delineated prior to commencing excavation |  |  |  |  |  |
|  | Temporary supports, batters & benches deeper than 1.2m have been reviewed by a competent engineer and assessment provided to Company Representative |  |  |  |  |  |
| Hold Point | Have items 1-5 been checked Approval to proceed with excavation |  |  |  |  |  |
|  | Confirm excavations for the construction of walls, footings, slabs, retaining walls or any structure shall extend a sufficient distance from the structure to permit placing and removal of forms, installation of services and inspection by the Engineer. |  |  |  |  |  |
|  | Excavations to be maintained and kept free of falling material and protected from water ingress. Measures to be taken to ensure excavation is left in safe condition, including barricading. Ramps or stairs in place to allow for safe access and egress for personnel and fauna. |  |  |  |  |  |
|  | Cover to reinforcement in footings and pedestals, unless otherwise noted: 75mm – cast against ground 50mm - cast in formwork |  |  |  |  |  |
|  | Excavation for Culverts installation shall be as per RVS-0000-SPE-Y-001Section 14. Drainage |  |  |  |  |  |
|  | Base of excavation inspected and free of unsuitable material |  |  |  |  |  |
|  | Base of excavation sufficiently compacted in accordance with RVS-0000-Y-001 unless otherwise approved: - 1 test per 50m2 or 3 tests per visit per layer (whichever requires the most) 300mm below surface level 95%MMDD and +/3% OMC |  |  |  |  |  |
|  | Set out base of foundation with extent and elevation of blinding prior to pouring concrete |  |  |  |  |  |
|  | Pour and level blinding or install underlay membrane as shown on documentation |  |  |  |  |  |
|  | Final survey checks to confirm excavation/ blinding level are in tolerance |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | CHECKED BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |