| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | Excavation permit in place |  |  |  |  |  |
|  | Define lots and set out works (Survey Data Report) |  |  |  |  |  |
|  | Place survey pegs to an offset: - The height of the embankment lift to ensure correct compaction and - Overfill sides to achieve hard butters after compaction and cut back to design line |  |  |  |  |  |
|  | Borrow pit management plan in place and approved |  |  |  |  |  |
|  | Prior to Placement in Lift No.1 - Embankment foundation approved by the Client (Ref. to C1.010) - Datum agreed with the Client |  |  |  |  |  |
|  | Prior to Placement of Consecutive Lift - Does previous lift conform? - Is the material sufficiently conditioned? |  |  |  |  |  |
| Hold Point | Have items 1-6 been checked |  |  |  |  |  |
|  | Material type and method of compaction been selected?  General Fill  General Rock Fill  Coarse Rock Fill |  |  |  |  |  |
|  | Type General Fill material, OMC ±3% with compaction MMDD of: - General Fill – 93% - Turkey’s Nest – 95% - Drainage Levee – 98% - Top 1m of embankment incl. capping – 95% |  |  |  |  |  |
|  | Type Rock Fill material to be trial pad result, developed Approved Method Specification (or Technical Query) Number ________________ |  |  |  |  |  |
|  | Trim batters as following: - Trim batter to achieve design line and slope as per drawings and specification - Clear all loose material from embankment |  |  |  |  |  |
| Hold Point | Approval of final trim, layer placement and construction tolerances. Has the multiple lift checklist been completed and signed off |  |  |  |  |  |

| Multiply Lift Checklist |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Material Type | Chainage (From – To) | Hold Point (Engineer signoff) | Lift Tested (Tick Box) | No. of Test | Date | Test Request # | Test Pass (Supervisor / Engineer) |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |