| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | Define lots and set out works (Survey set out and pickup) |  |  |  |  |  |
|  | Underlying Lot is fully compliant and signed off |  |  |  |  |  |
|  | Expose underground services/structures by hand or vacuum truck |  |  |  |  |  |
|  | Foundation surface inspected and classified as suitable or unsuitable with approval from Client Representative Classification: Rock/ Common/ Unsuitable |  |  |  |  |  |
| Hold Point | Have items 1-3 been checked |  |  |  |  |  |
|  | Embankment foundation in Common, Fill Depth > 500mm: Prepare to test 200mm below surface level: - Test for MMDD=92% and - Optimum Moisture Content OMC +/- 3% |  |  |  |  |  |
|  | Embankment foundation in Common, Fill Depth < 500mm: Prepare to test 200mm below surface level: - Test for MMDD=95 % and - Optimum Moisture Content OMC +/- 3% |  |  |  |  |  |
|  | Embankment foundation for Structure, Prepare to test 300mm below surface level: - Test for MMDD=95 % and - Optimum Moisture Content OMC +/- 3% |  |  |  |  |  |
|  | Embankment foundation for Trench, Prepare to test 200mm below surface level: - Test for MMDD=90 % and - Optimum Moisture Content OMC +/- 3% |  |  |  |  |  |
|  | Embankment foundation for Turkey’s Nest, (Borrow pit or Excavated): - Test for MMDD=95 % and Optimum Moisture Content OMC +/- 3% |  |  |  |  |  |
|  | Embankment foundation in Rock: - Clean loose material and make suitable - for construction prior to fill - No water or rolling required |  |  |  |  |  |
| Unstable Materials |  |  |  |  |  |  |
|  | Unsuitable surface has been inspected |  |  |  |  |  |
|  | Verify That Unsuitable Materials: - Remove to extent and depth as directed by Client or where suitable material is reached. - Dispose to nominated spoil location and, - Survey the excavation (Survey Report) |  |  |  |  |  |
|  | Base of excavation inspected if suitable for material placement. |  |  |  |  |  |
| Hold Point | Have items 11-13 been checked |  |  |  |  |  |
|  | Additional excavation required if suitable bearing soil is not reached at design depth as directed by the Client: Prior for backfilling, area has been: - Reinspected and surveyed - Unsuitable material removed and agreed by survey (Survey Data Report) |  |  |  |  |  |
|  | Disposal of unsuitable materials in approved spoil locations |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |