| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| Marked up drawing attached: |  |
| --- | --- |
| Estimated Volume |  |
| Fill Material Tested (Attach test report with ITR) |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  |  |  |  |
|  | Set out works |  |  |  |
|  | Specified concrete curing time achieved |  |  |  |
|  | Structural fill material compliant, all backfill material is structural fill as per 9.3 of RVS-0000SPE-Y-001. (test report) |  |  |  |
| Hold Point | Have items 2-3 been checked Approval to proceed with backfill |  |  |  |
|  | Backfill within one meter of concrete structures not containing particles exceeding 75mm. |  |  |  |
|  | Backfill placed in contact with waterproofing membranes shall be sand fill as per 9.4 of RVS-0000-SPE-Y-001. or selected approved materials and all necessary precautions shall be taken to protect the membrane or coatings. |  |  |  |
|  | Backfill placement and compaction as per 10.6 and 13 of RVS-0000-SPE-Y-001. Structural Fill: - Loose layers of uniform thickness not exceeding 300mm - Compaction ≥98% MMDD - Moisture content ±3% of OMC - DDR & MC – 1 test per 50m2 or 3 tests per lot - (whichever requires the most tests) - PSD, PI, PL, LL – 1 test per 2500m3 - CBR Strength – 1 test per 5000m3 Sand Fill: - Compaction ≥95% MMDD - Moisture content ±3% of OMC - DDR & MC – 1 test per 50m2 or 3 tests per lot - (whichever requires the most tests) - PSD, PI, PL, LL – 1 test per 2500m3 - CBR Strength – 1 test per 5000m3 |  |  |  |
|  | Final survey checks to confirm final level tolerance |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. I hereby confirm all Inspection Test Plan (ITP) Hold and Witness points have been completed by the Contractor & Engineer |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |