| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | Define lots and set out works for subgrade as per RVS-0000-SPE-Y-001, Section 17.2.2 (Survey Data Report) |  |  |  |  |  |
|  | Underlying lot conforms: - Ensure underlying lot conforms, all outstanding items are closed and signed off - Underlying lot in acceptable state to start work. |  |  |  |  |  |
|  | Confirm subgrade material conforms to specifications. Test Reports for: - Particle Size Distribution, Liquid Limit, Plasticity Index, Plastic Limit, Linear Shrinkage & CBR Strength |  |  |  |  |  |
|  | Prepare subgrade layer to: - 150mm below subgrade surface for embankment fill or - 300mm when subgrade is at foundation level. |  |  |  |  |  |
|  | Subgrade compacted to: - MMDD of 95% and OMC of ± 3% OMC. - Where rock is present in cutting subgrade, no subgrade preparation is required. |  |  |  |  |  |
| Hold Point | Have items 1-5 been checked |  |  |  |  |  |
|  | Subgrade Survey Frequency as per RVS-0000-SPE-Y-001, Section 17.3: - 3points equally spaced apart per 10m strip (at the two edges and one in the middle every 20m) |  |  |  |  |  |
|  | Subgrade Width Tolerance: - Deviation of < 150mm at any point along batter |  |  |  |  |  |
|  | Subgrade Surface Level: - General Earthworks +0mm / -15mm (Deviation from a 3 m straight edge laid along the finished surface <15mm - Beneath cast in-situ concrete, pads, and hardstand areas +20mm / - 50mm - Building pads +20 / -20mm (Deviation from a 3 m straight edge laid along the finished surface <30mm) |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |