| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| ITP Ref / No.: |  |
| --- | --- |
| Location / Area / Lot |  |

| Item No | Description | Completed by |  | Accepted by |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| ITEMS TO BE CHECKED PRIOR TO APPROVAL |  | Initials | Date | Initials | Date |  |
|  | Approve previous layer - Basecourse accepted (Survey Data Report) |  |  |  |  |  |
|  | Define lots and set out works |  |  |  |  |  |
| Hold Point | Final Design: Prior to the application of any sprayed seal, the Contractor shall provide the Superintendent with evidence of the application of appropriate design methods. (Spec. Bituminous Surfacing 503.23.03 Main Road WA) |  |  |  |  |  |
| Hold Point | Bitumen Compliance: Prior to the on-site delivery bitumen, the Contractor shall demonstrate compliance with the bitumen property requirements for each batch of bitumen prior to the bitumen being used in the contract. Certificate of Compliance (Spec 503.42.01.3 Main Road WA) |  |  |  |  |  |
| Hold Point | Aggregate Compliance: Prior to the on-site delivery of crushed aggregate, the Contractor shall provide aggregate test certificate for approval. (Spec. 503.16.01.5 Main Road WA) (Spec 503.54 & 503.55 and Table 503.4 & 503.5). Test frequency to comply with Spec. 201 |  |  |  |  |  |
|  | Binder Storage & Disposal Sites: The Contractor shall select suitable sites for bulk storage of bituminous materials and notify the Engineer at least five (5) days in advance of establishment or use. (Spec 503.26.01.1: Nominate Binder Storage Sites) |  |  |  |  |  |
|  | Audit Samples: Samples to be taken for audit purposes during the project. (Spec 503.06.02) |  |  |  |  |  |
|  | Aggregate Stockpile Sites Prior to the stockpiling of aggregate, the Contractor shall nominate to the Engineer the proposed location of the stockpile sites and provide a management plan for the sites. (Spec. Spec 503.26.01.3) |  |  |  |  |  |
| Hold Point | Calibration Sprayer: Prior to the use of the sprayer on the Works, the Contractor shall make available to the Engineer the calibration certificate for the sprayer. (Spec 503.33.01.3: Calibration certificate Sprayer conformity, nozzles, rollers, brooms etc. as detailed in Spec 503.33) |  |  |  |  |  |
|  | Surface Preparation: The Contractor shall make sure that any required sweeping and or / necessary repairs of the Basecourse surface have been completed, and the surface is suitable to receive the subsequent bituminous treatment. (Minimum 1 hour prior to application) |  |  |  |  |  |
|  | Protective paper: heavy duty paper shall be used for all start, finish and taper operations on sprayed sealing works. (Spec 511.19) |  |  |  |  |  |
|  | Traffic Control: Approved Traffic Control Plan will be in place prior to implementing any coat (particularly temporary speed restrictions). |  |  |  |  |  |
|  | Application of Bitumen Emulsion Primer Seal: - The surface to be sealed shall be dry and no binder shall be applied during wet or rainy conditions, or when adverse weather conditions may prevail at any time during such work - Pavement temp between 10-40C - Binder spraying temperature between 35 and 50C (Spec 503.45.02) |  |  |  |  |  |
|  | Binder & Emulsion Preparation: - Bitumen, cutback bitumen or 5% rubber binder and other constituents, as detailed in Annexure 503C shall be mixed by circulation in the mechanical sprayer for not less than fifteen minutes immediately prior to application or such longer periods as may be necessary to ensure a uniform and homogeneous mixture. - Bitumen emulsion shall be mixed by circulation in the mechanical sprayer for not less than ten minutes or such longer period as may be necessary to ensure a uniform and homogeneous mixture (Spec 503.41.01 Hot Binders and Spec 503.41.02 Bitumen emulsion) |  |  |  |  |  |
|  | Primer seal & Seal Aggregate preparation: Aggregate to be prepared in accordance with Spec 503.34. |  |  |  |  |  |
|  | Binder Application: - Binder shall not be applied until the Contractor has sufficient spreader trucks to comply with 503.45.03(1a) at the location of the area to be sprayed. - All rollers shall be at the location of the area to be sprayed at the time of spraying. (Spec 503.45.02) |  |  |  |  |  |
|  | Cover Aggregate Application Rate: Aggregate Type and Spread Rate to be reported as required in Annex. 503.B (Spec 503.44.04.1, 2 & 3) |  |  |  |  |  |
|  | 1st Coat Aggregate: - The sprayed binder shall be totally covered with aggregate within 8 minutes of the application of the binder - The length of spray runs shall be limited to comply with this requirement (Spraying tolerances as per Spec 503.42.04.09 and 503.45.03.1) |  |  |  |  |  |
|  | Emulsion Application – 2nd Coat: In double/double seals, the binder for the second coat shall not be applied until the binder in the first coat has completely broken and cured to form a stable seal leaving no water in the binder (Visual Inspection) |  |  |  |  |  |
|  | 2nd Coat Aggregate: Second coat shall be spread and rolled within 15 minutes of the application of the second binder coat (Spec 503.45.03.1) |  |  |  |  |  |
|  | Overlap Compliance: The second coat seal shall overlap the larger aggregate of the first coat as detailed (Spec 503.42.04.9) (Visual Inspection) |  |  |  |  |  |
|  | Rolling: Min 12 passes, as per Annex. Table 503 C9 (Spec 503.44.02) |  |  |  |  |  |
|  | Sweeping: - Surface swept of loose excess aggregate after rolling at the completion - If required following 7 days sweeping shall be carried out to avoid loose stones - Install “loose stones” signs (Spec 503.45.04:) (Site Inspection) |  |  |  |  |  |
|  | Non-conforming results / Punch list: - Within 60 days of completion, when defect arise in a seal or re seal the MRWA / super’s reps may direct the contractor to repair or replace any defective sections. - Non-conformance in binder application, where sprayed work does not meet the tolerances given in - For seals and bitumen emulsions primer seals the Contractor shall apply corrective actions - No payment shall be made for binder sprayed outside the 50 mm margin (Spec 503.42.04 and Table 503.5) |  |  |  |  |  |
|  | NCR / Punch List items closed out and work completed: - Specifications - Red Line drawings - Survey Data Reports - Quality Certificates submitted |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (NO PUNCHLIST ITEMS TO BE INCLUDED HERE) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |