| Tag No. «Name» - «Description» |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |  |
| Cable Type: | «CableType» |  | Cable Rating: |  | «CableVoltage» |
| Cores: | «CableNumConductors» | Cable Size: |  | «CableSize» |  |
| From: | «CableFrom» | To: |  | «CableTo» |  |
| Drawing No. | «DocumentList» | Task No. |  | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |  |

| Item | Description of Check | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm that the cable is of correct size, type and colour as stipulated in the cable schedule. |  |  |  |
|  | Confirm cable identification markings are in place. |  |  |  |
|  | Check cable and confirm there is no mechanical damage. |  |  |  |
|  | Confirm cable route and to and from equipment locations are correct as per the cable schedule/ routing layout. |  |  |  |
|  | Confirm cable cores are numbered correctly at both ends as per cable schedule. |  |  |  |
|  | Confirm that ferrule numbers are in place where required and are correct. |  |  |  |
|  | Confirm segregation between the cables with different voltage levels is maintained as per the design. |  |  |  |
|  | Confirm all terminations are tight and where torque settings are required, that correct values have been obtained and the connection has been marked to indicate completion. |  |  |  |
|  | Confirm that earth cable has been terminated in accordance with schematic. |  |  |  |
|  | Confirm cable support and enclosure systems are satisfactory and secure. |  |  |  |
|  | Confirm that the minimum cable bending radius has not been exceeded. |  |  |  |
|  | Confirm cable installed as per the cable layout drawings and project specs. |  |  |  |
|  | Confirm correct cable glands for cable type, glands are screwed as per manufacturer’s instructions. |  |  |  |
|  | Confirm cable termination to ensure all lugs/pins are crimped correctly, nuts, washers and spring washers are in place and terminals are clean and free of dust. |  |  |  |
|  | Confirm that all spare cores are terminated and are earthed at one end. |  |  |  |

| INSULATION / CONTINUITY TESTS |  |  |
| --- | --- | --- |
| Test Equipment: | Serial No.: | Calibration Date: |
| Test Equipment: | Serial No.: | Calibration Date: |

| Continuity & Insulation Resistance Tests | Earth Continuity Result Ohms (Ω) & Point to Point | Insulation Resistance 250 VDC for 1 min Result (MΩ) | Pass | Fail | Comment |
| --- | --- | --- | --- | --- | --- |
| Earth | Ω |  |  |  |  |
| Screen | Ω |  |  |  |  |
| Cores to E |  | MΩ |  |  |  |
| Core 1 |  |  |  |  |  |
| Core 2 |  |  |  |  |  |
| Core 3 |  |  |  |  |  |
| Core 4 |  |  |  |  |  |
| Core 5 |  |  |  |  |  |
| Core 6 |  |  |  |  |  |
| Core 7 |  |  |  |  |  |
| Core 8 |  |  |  |  |  |
| Core 9 |  |  |  |  |  |
| Core 10 |  |  |  |  |  |
| Core 11 |  |  |  |  |  |
| Core 12 |  |  |  |  |  |
| Core 13 |  |  |  |  |  |
| Core 14 |  |  |  |  |  |
| Core 15 |  |  |  |  |  |
| Core 16 |  |  |  |  |  |
| Core 17 |  |  |  |  |  |
| Core 18 |  |  |  |  |  |
| Core 19 |  |  |  |  |  |
| Core 20 |  |  |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm terminals are correctly re-tensioned. |  |  |  |
|  | Confirm circuit left safe and ready for energisation. |  |  |  |
|  | Confirm As Built changes marked up on drawings/schedules. |  |  |  |
|  | Confirm enclosures are reinstated correctly, including gaskets, seals and fixings. |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | Initial | N/A | Comments |
|  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |
|  | This form must be completed by an Authorised Electrical Worker. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| EW Licence No.: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |