| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm Earth Bars conform to specification and design drawings. |  |  |  |
|  | Confirm MEN Point and link connections are installed, torqued to spec and free from corrosion. |  |  |  |
|  | Confirm Earth Cable connections are installed, torqued to spec, secured to the Earth Bar and free from corrosion. |  |  |  |
|  | Confirm that all Neutral cables are the correct size and correctly labelled/ identified in accordance with cable schedule or layout drawing. |  |  |  |
|  | Confirm that all earth cables are the correct size and correctly labelled/ identified in accordance with cable schedule. |  |  |  |
|  | Confirm that all equipotential bonding earth cables are the correct size and correctly labelled/ identified in accordance with the design. |  |  |  |
|  | Confirm that equipotential bonding connections of structure and equipment are clean, tight, secure and coated with protective coating. |  |  |  |
| Note: All tests below must be supported by an attached grid and stake layout drawing or Sketch to support the location and identification of test points |  |  |  |  |

| MEN CONTINUITY & CONTACT RESISTANCE MEASUREMENTS |  |  |  |
| --- | --- | --- | --- |
| Test Equipment: | Serial No.: | Calibration Date: |
| Test Points | Point to Point Check (Initial Pass/Fail) | Continuity/ Contact Resistance – Result Ω |
| TX Star - NER |  | Ω µΩ |
| Across NER |  | Ω µΩ |
| NER – E Bar |  | Ω µΩ |
| TX STAR – E Bar |  | Ω µΩ |
| MEN-E Link |  | Ω µΩ |
| MEN Other: |  | Ω µΩ |
|  | Point to Point Confirmed Y N | Continuity Confirmed Pass Fail |

| EARTH BAR & EQUIPOTENTIAL BONDING CONTINUITY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Equipment: |  |  | Serial No.: |  |  | Calibration Date: |  |
| Attach layout drawing to indicate test points |  |  |  |  |  |  |  |
| Test Point A ID | Test Point B ID | Resistance (Ω) |  | Test Point A ID | Test Point B ID |  | Resistance (Ω) |
| Point to Point Confirmed Y N |  |  |  | Continuity Compliance Pass Fail |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | Initial | N/A | Comments |
|  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |
|  | This form must be completed by an Authorised Electrical Worker. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| EW Licence No.: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |