| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. | «OEMPartNumber» |  |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. |  | «TaskName» |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| This check sheet applies to Three different installation types of the Belt Rip Switch. Section 1 is applicable to all types of installations. Make the correct selection on other sections based on the applicable installation type below. |  |  |  |  |
| Section 1 - Common to all Belt Rip Detector/Switch installations |  |  |  |  |
| PHYSICAL CHECKS |  |  |  |  |
|  | Mounting is firm, IP rating not compromised and not subject to vibration. |  |  |  |
|  | IP rating is correct. |  |  |  |
|  | Labelling is correct |  |  |  |
|  | Device location is correct as per conveyor layout |  |  |  |
|  | Surface finish is undamaged. |  |  |  |
|  | Cable service loop is sufficient for range of adjustment. |  |  |  |
|  | All gaskets and seals are intact. |  |  |  |
|  | Cable glands are correct and shrouded if applicable. |  |  |  |
|  | Bootlace pins or approved lugs are used on all wires and are firmly crimped. |  |  |  |
|  | Wiring looms are neat. |  |  |  |
|  | All wires are ferruled and ferrules are correct. |  |  |  |
|  | All spare cores are terminated and ferruled as spare. |  |  |  |
|  | Terminals are numbered, firmly tightened with one wire per terminal. |  |  |  |
|  | Check terminations to ensure that all nuts, washers and spring washers are in place. |  |  |  |
|  | Inspect internal equipment for defects or damage. |  |  |  |
|  | Equipment clean and free from debris internally and externally. |  |  |  |
|  | Circuit left safe and ready for energisation. |  |  |  |
| Section 2 – Applicable to Through Belt Type Belt Rip Detector (Type 1) |  |  |  |  |
|  | Type 1 - As per standard drawing MA-0000-M-1048 |  |  |  |
|  | Device installation and orientation is correct as per standard drawing MA-0000-M-1048. |  |  |  |
|  | Confirm installation, orientation and alignment is not impacted by adjacent structural steel, removable guards or other mechanical equipment. |  |  |  |
|  | Confirm the installation bracket is in accordance with the Belt Angle profile and provides for adjustment. |  |  |  |
|  | Confirm Instrument rip wire is correctly routed through all eye bolts, fixed guides and spring guides |  |  |  |
|  | Confirm the rip wire is free of any wear or damage and rope grips are tensioned. |  |  |  |
|  | Confirm Instrument rip wire is correctly aligned with the belt profile with 25mm of separation maintained over the entire belt. |  |  |  |
|  | Confirm the rip wire turnbuckle tension and insertion mounting of the rip switch plug is satisfactory. |  |  |  |
|  | Confirm the rip switch plug tether is of adequate length and does not interfere with belt or structure when in the disengaged state. |  |  |  |
| Section 3 – Applicable to Tail Pulley String Type Belt Rip Detector (Type 2) |  |  |  |  |
|  | Type 2 - As per standard drawing MA-0000-M-1050 |  |  |  |
|  | Device location and orientation is correct as per standard drawing MA-0000-M-1050 |  |  |  |
|  | Confirm installation, orientation and alignment is not impacted by adjacent structural steel, removable guards or other mechanical equipment. |  |  |  |
|  | Confirm rip wire and switch mounting allows suitable mechanical adjustment for switch and trip wire lateral alignment. |  |  |  |
|  | Confirm the rip wire is free of any wear or damage and rope grips are tensioned. |  |  |  |
|  | Confirm Instrument rip wire is correctly aligned with the belt pulley profile with 25mm of separation maintained over the entire belt. |  |  |  |
|  | Confirm the rip wire turnbuckle tension and insertion mounting of the rip switch plug is satisfactory. |  |  |  |
|  | Confirm the rip switch plug tether is of adequate length and does not interfere with belt or structure when in the disengaged state. |  |  |  |
| Section 4 – Applicable to Pan Type Belt Rip Detector (Type 3) |  |  |  |  |
|  | Type 3 - As per standard drawing MA-0000-M-1049 |  |  |  |
|  | Device location and orientation is correct as per standard drawing MA-0000-M-1049. |  |  |  |
|  | Confirm the Pan and pivot mechanism is securely fastened and the Pan pivots through the full range of required movement and is not obstructed by conveyor structure. |  |  |  |
|  | Load the Pan with 4.5kG and adjust the counterweights to confirm RIP detector Operation. Secure the counterweights |  |  |  |
|  | Confirm Sensor mounting allows suitable mechanical adjustment for alignment and orientation with regards to its actuator plate flag. |  |  |  |
|  | Confirm Sensor mounting allows for 20mm separation between the sensor face and actuator plate in the operating position and greater than 100mm of separation in the tripped position. . |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Item |  |  |  | Initial | N/A | Comments |  |  |
| 1 | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |  |  |
| 2 | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |  |  |  |  |  |
| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |  |  |  |  |
| COMPLETED |  |  |  |  |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  |  |  |  | EPCM ACCEPTED BY: |  |
| Signature: |  |  | Signature: |  |  |  |  | Signature: |  |
| Name: |  |  | Name: |  |  |  |  | Name: |  |
| Licence No: |  |  | Title: |  |  |  |  | Title: |  |
| Date: |  |  | Date: |  |  |  |  | Date: |  |