| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| GENERAL DATA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Schematic: |  |  | Termination: |  |  |  |  |  |  |
| Valve Duty | FCV | FV |  | MV |  | DV | IV | AV | CV |
|  | SRV | SV |  | WCV |  |  |  |  |  |
| Actuator: | Motor | Pneumatic |  | Hydraulic |  | Geared |  |  |  |
| Actuator Method | Electro/Mech | Mechanical |  | Cylinder |  | Solenoid | Diaphragm |  |  |
| Signal | 4-20mA | Digital |  | Other |  |  |  |  |  |
| Operation | ON/OFF | Modulating |  | Position |  |  |  |  |  |
| Fail Safe Mode | FO | FC |  | FL |  |  |  |  |  |
| Instrument Supply Voltage |  |  |  |  | Instrument Control Voltage |  |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | This ITR is applicable to installed instrument Control Valves which are Motorised, Pneumatic, Hydraulic, ON/OFF or Modulating. If the instrument incorporates an external control panel then an Instrument control panel ITR is applied. |  |  |  |
|  | Section 1 is applicable to the valve instrument installation and instrument terminal box terminations |  |  |  |
|  | Confirm instrument type (General Data) is in accordance with the instrument data sheet |  |  |  |
|  | Confirm instrument location, installation and orientation is correct as per the design drawings. |  |  |  |
|  | Confirm the terminal box is free from damage, rust and contamination internally and externally. |  |  |  |
|  | Confirm internal and external labelling and tagging is correct, legible, of correct material and securely fitted to the equipment. |  |  |  |
|  | Confirm sunshade is fitted to installations exposed to direct sunlight, as applicable. |  |  |  |
|  | Check Instrument mountings to ensure that all nuts, washers and spring washers are in place and tensioned correctly. |  |  |  |
|  | Confirm all unused cable entries are plugged with suitable blanks. |  |  |  |
|  | Confirm cable service loops are adequate and mechanically protected where applicable. |  |  |  |
|  | Confirm all cable terminations are correct as per the drawings |  |  |  |
|  | All spare cores are terminated and ferruled as spare. |  |  |  |
|  | Terminals are numbered, firmly tightened with one wire per terminal. |  |  |  |
|  | Wiring looms are neat with ducting covers fitted. |  |  |  |
|  | Confirm instrument earth bonding is connected, tightened, IE and PE earth bars are in place and labelled correctly as per applicable drawings. |  |  |  |
|  | Confirm ELV and LV segregation, terminal covers and labelling is maintained between instrument control and terminal box power terminations. |  |  |  |
|  | Confirm the circuit is left safe and ready for energisation. |  |  |  |
|  | INSTRUMENT AIR | N/A |  |  |
|  | Confirm Instrument Air tubing, bends and fittings are to design specification. |  |  |  |
|  | Confirm Instrument Air tubing has been installed to design and specification with adequate supports and mechanical protection where required. |  |  |  |
|  | Confirm Instrument air, isolation double block and bleed valves are installed, accessible and labeled. |  |  |  |
|  | Inspect instrument factory tubing and fittings for transport damage and tension. |  |  |  |
|  | If applicable, confirm the instrument regulator/air/oil separator is installed as per design, the collector is clean and isolation vent and drain lines clean and unimpeded. |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | Initial | N/A | Comments |
| 1 | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |
| 2 | This form must be completed by an Authorised Electrical Worker. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Licence No: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |