| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm tag nameplate (or stamping) is clear, legible and securely fitted to the equipment. |  |  |  |
|  | Confirm all components are free from damage, contamination or corrosion. |  |  |  |
|  | Confirm device/ sensor positioning, alignment and orientation is correct with regards to the process that will be measured/ monitored. |  |  |  |
|  | Confirm device/sensor mounting allows suitable mechanical adjustment for alignment and orientation with regards to its process interface. |  |  |  |
|  | Confirm cable service loop is sufficient for range of adjustment. |  |  |  |
|  | Confirm cable glands are correct and tensioned. |  |  |  |
|  | Confirm all unused cable entries and process tappings are plugged with suitable blanks. |  |  |  |
|  | Confirm bootlace pins or approved lugs are used on all wires and are firmly crimped. |  |  |  |
|  | Confirm wiring looms are neat. |  |  |  |
|  | Confirm terminals are numbered, firmly tightened with one wire per terminal. |  |  |  |
|  | Confirm all wires are ferruled and ferrules are correct. |  |  |  |
|  | Confirm all spare cores are terminated and ferruled as spare. |  |  |  |
|  | Confirm equipment earth bonding is connected as per applicable drawings. |  |  |  |
|  | Confirm equipment clean and free from debris internally and externally. |  |  |  |
|  | Confirm all gaskets and seals are intact. |  |  |  |
|  | Confirm device is left safe and ready for energisation. |  |  |  |

| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | Initial | N/A | Comments |
| 1 | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |
| 2 | This form must be completed by an Authorised Electrical Worker. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| EW Licence No: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |