| Tag No. «Name» - «Description» |  |  |  |
| --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |
| Additional Information: | «Comments» |  |  |

| Item | Description |  |  |  |  | OK | N/A | Punch |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tag nameplate (or stamping) is clear, legible, of correct material and securely fitted to the equipment. |  |  |  |  |  |  |  |
|  | Inspect all components for damage, contamination, corrosion or poor quality. |  |  |  |  |  |  |  |
|  | Confirm installation location and instrument type is in accordance with the design. |  |  |  |  |  |  |  |
|  | If applicable confirm device mounting allows suitable mechanical adjustment for alignment and orientation. |  |  |  |  |  |  |  |
|  | If applicable confirm sunshade is fitted to installations exposed to direct sunlight. |  |  |  |  |  |  |  |
|  | Inspect the field location of the installation to ensure the indicator, siren of traffic light is accessible for maintenance. |  |  |  |  |  |  |  |
|  | Check mountings to ensure that all nuts, washers and spring washers are in place and tensioned correctly. |  |  |  |  |  |  |  |
|  | Confirm all unused cable entries are plugged with suitable blanks. |  |  |  |  |  |  |  |
|  | Confirm all cable terminations are correct as per the drawings |  |  |  |  |  |  |  |
|  | Confirm all spare cores are terminated and ferruled as spare. |  |  |  |  |  |  |  |
|  | Terminals are numbered, firmly tightened with one wire per terminal. |  |  |  |  |  |  |  |
|  | Confirm equipment earth bonding is connected as per applicable drawings. |  |  |  |  |  |  |  |
|  | Circuit left safe and ready for energisation. |  |  |  |  |  |  |  |
| COMPLETIONS & CLIENT ACCEPTANCE REQUIREMENTS – CONTRACTOR CHECK LIST |  |  |  |  |  |  |  |  |
|  |  | Item | Initial | N/A | Comments |  |  |  |
| 1 |  | All site modifications correctly marked up on master drawings as per project procedures. |  |  |  |  |  |  |
| 2 |  | All Punchlist items noted on a separate Punchlist form and submitted for. |  |  |  |  |  |  |
| 3 |  | This form must be completed by an Authorised Electrical Worker. |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punch List Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Licence No: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |