| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm materials of construction are as per project specifications, manufacturers specifications and design drawings. |  |  |  |
|  | Confirm support structure has been accepted. |  |  |  |
|  | Confirm all connection bolts to the above silo have been fitted and torqued and no distortion of the flange has taken place. |  |  |  |
|  | Confirm alignment arrows on the upper and lower sections align and the Bin Activator Seal has been installed as per MJ-2591-M-50025. |  |  |  |
|  | Confirm vibrator motor has been installed and all bolts torqued as per MJ-2591-M-50025. |  |  |  |
|  | Confirm isolation hangers, isolation bushes and all fixings have been installed, checked and torqued. |  |  |  |
|  | Confirm discharge slide gate has been installed, all nuts bolts and washers have been installed and torqued to vendor specifications and slide gate moves freely to the open and closed positions without binding or undue force. |  |  |  |
|  | Confirm the connection between the Activator discharge and the Dry Flocculant Feeder is complete and tight. |  |  |  |
|  | Confirm all transport plates have been removed. |  |  |  |
|  | Confirm nameplate has been attached and is clearly visible. |  |  |  |
|  | Confirm all debris and waste material have been removed. |  |  |  |
|  | Confirm lifting lugs are attached, stamped and fit for purpose as per project specification MA-0000-M-1124. |  |  |  |
|  | Confirm surface protection has been applied as per SS-P118 and MJ-2591-M-50025 and any touch up painting has been completed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |