| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm installation is checked against Vendor Drawings and Project Specifications. |  |  |  |
|  | Confirm the agitator blade hub is firmly affixed to the agitator shaft. |  |  |  |
|  | Confirm each agitator blade is firmly affixed to the agitator blade hub. |  |  |  |
|  | Confirm all bolted connections are of the correct grade and tensioned correctly. |  |  |  |
|  | Confirm the agitator/mixer assembly is firmly attached to the mounting plate as per vendor recommendations. |  |  |  |
|  | Confirm the agitator/mixer assembly is attached to the tank/structure as per vendor recommendations. |  |  |  |
|  | Confirm the drive motor is connected to the agitator/mixer as per vendor recommendations. |  |  |  |
|  | Confirm the direction of rotation is marked on the agitator/mixer housing and is clearly visible. |  |  |  |
|  | Confirm the agitator gearbox is filled to the correct level with the correct grade of oil as per vendor specifications. Oil Type _________________ Volume __________________ |  |  |  |
|  | Confirm all temporary transport Shaft Locks and Bracing removed as per vendor specifications. |  |  |  |
|  | Confirm all identification tags and safety signage are installed as per design drawings and project specifications. |  |  |  |
|  | Confirm all surface protection and touch up painting has been completed to SS-P118. |  |  |  |
|  | Confirm all guards are in place. |  |  |  |
|  | Confirm Vendor stage 1 Installation Sheet (M1.099) has been completed, witnessed and attached. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |