| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm a thorough visual inspection of all components and all are free from damage including ancillary items i.e. lubrication components. |  |  |  |
|  | Confirm Apron Feeder installation is as per design drawings. |  |  |  |
|  | Confirm all bearing transport protection removed. |  |  |  |
|  | Confirm pan bolt tension checked as per value indicated in vendor IOM 5% check conducted. |  |  |  |
|  | Confirm 5% check of guide roller bolts. If any found loose, alignment check to be undertaken. |  |  |  |
|  | Confirm inspection of segmented drive sprocket and confirm bolts are torqued correctly as per OEM requirements. |  |  |  |
|  | Confirm remaining bolted connections checked for correct tension 5%. |  |  |  |
|  | Confirm all hoses & cables are secured and clear from rotating elements. |  |  |  |
|  | Confirm clearance between pans & skirts as per vendor IOM. Gap _____mm |  |  |  |
|  | Confirm all rubber curtains and skirts installed as per design. |  |  |  |
|  | Confirm all rotating elements are free from obstruction. |  |  |  |
|  | Confirm all lubrication lines have been connected, pigged, purged and labelled. |  |  |  |
|  | Record the Grease type: _______________________ |  |  |  |
|  | Confirm all hydraulic and lubrication tanks and lines have been flushed, pigged and purged. |  |  |  |
|  | Confirm all first fills have been completed, samples taken and results attached. |  |  |  |
|  | Confirm chain tension is as per vendor IOM, all hydraulic systems are installed, protected and functional. |  |  |  |
|  | Confirm take up bearing slides have been lubricated as per OEM Spec. |  |  |  |
|  | Confirm both sliding bearings have been extended equal distances from jacking pads. |  |  |  |
|  | Confirm head chute clear of pulley and pans. |  |  |  |
|  | Confirm all guarding is in place & secure. |  |  |  |
|  | Confirm safety lanyards installed complete with signage. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm vendor documentation and ITR (M1.099) has been reviewed, approved and attached. |  |  |  |
|  | Survey completed and approved and survey report attached. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |