| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

If any items are purely vendor supplied, aligned and sealed units with no chance of movement during transport and installation, please attach the relevant Factory Acceptance Test sheets or other documentation to verify checks.

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and confirm they are free from damage. |  |  |  |
|  | Confirm bearing housings clearly identified as fixed or floating. |  |  |  |
|  | Confirm surface/ corrosion protection removed from bearing housing mounting face prior to install. |  |  |  |
|  | Confirm hold down bolts are of the correct length, grade and tension and adjusting bolts locked and Denso taped. |  |  |  |
|  | Confirm bearing cap bolts tensioned to manufacturer’s specification. |  |  |  |
|  | Confirm housings are aligned such that the radial gap measured between the labyrinth seals at any two diametrically opposite points does not vary by more than 0.5mm. Gap to be confirmed and recorded in the below table. |  |  |  |
|  | Confirm bearing/s rotates freely by hand. |  |  |  |
|  | Confirm shims are installed in line with manufacturer and project standards. |  |  |  |
|  | Confirm remote grease lines installed correctly, flushed/pigged and purged. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| LABYRINTH SEAL CLEARANCES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INNER- FIXED |  |  | OUTER- FIXED |  |  | INNER- FLOATING |  | OUTER- FLOATING |  |
| A B C D |  |  | E F G H |  |  | I J K L |  | M N O P |  |
|  | Attach marked up drawing to show location of bearing numbers. |  |  |  |  |  |  |  |  |
|  | BRG 1 | BRG 2 |  | BRG 3 | BRG 4 | BRG 5 | BRG 6 | BRG 7 | BRG 8 |
| A |  |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |  |  |
| F |  |  |  |  |  |  |  |  |  |
| G |  |  |  |  |  |  |  |  |  |
| H |  |  |  |  |  |  |  |  |  |
| I |  |  |  |  |  |  |  |  |  |
| J |  |  |  |  |  |  |  |  |  |
| K |  |  |  |  |  |  |  |  |  |
| L |  |  |  |  |  |  |  |  |  |
| M |  |  |  |  |  |  |  |  |  |
| N |  |  |  |  |  |  |  |  |  |
| O |  |  |  |  |  |  |  |  |  |
| P |  |  |  |  |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |