| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm Belt Feeder installation is correct as per vendor drawings. |  |  |  |
|  | Confirm Belt Feeder installation as per design drawing work points and survey report is attached. |  |  |  |
|  | Confirm a thorough visual inspection has been undertaken for any transport damage including lubrication components. |  |  |  |
|  | Confirm all bearing transport protection has been removed. |  |  |  |
|  | Confirm a 10% check of idler bolts has been carried out. If any found loose, an idler alignment check is to be undertaken. |  |  |  |
|  | Confirm all remaining bolted connections checked for correct tension, 10% check minimum. |  |  |  |
|  | Confirm feeder drive and torque arm have been installed as per vender specification. (M1-025 – Drive Assembly completed) |  |  |  |
|  | Confirm all bearings have been inspected and lubricated as per manufacturers specification. (M1-014 Bearing ITR completed) |  |  |  |
|  | Confirm all auto lubrication systems have been installed to the belt feeder. |  |  |  |
|  | Confirm all piping connected to grease unit has the correct pressure rating. |  |  |  |
|  | Confirm all lubrication hoses, nipples and connections are tight and any points where chafing is possible have been protected. |  |  |  |
|  | Confirm all lubrication lines have been connected & lube tank filled with grease and flushed/pigged. |  |  |  |
|  | Record grease type: __________ |  |  |  |
|  | Confirm belt tension is as per vendor IOM and Denso tape has been applied to take up screw threads. |  |  |  |
|  | Confirm clearance between hopper liner & skirt are as per vendor IOM and design drawings. |  |  |  |
|  | Confirm all rotating elements are free from obstruction. |  |  |  |
|  | Confirm feeder locking pins can be engaged & disengaged by use of hand pump. |  |  |  |
|  | Confirm safety lanyards installed complete with signage. |  |  |  |
|  | Confirm all guarding is in place & secure. |  |  |  |
|  | Confirm Vendor stage 1 Installation Sheet (M1.099) has been completed, witnessed and attached. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |