| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm all transport protection & bracing has been removed. |  |  |  |
|  | Confirm the belt reeler is thoroughly checked for any transport damage. |  |  |  |
|  | Confirm installation is in accordance with relevant drawings. |  |  |  |
|  | - Confirm all bolted connections are of the correct length, grade and tensioned correctly. |  |  |  |
|  | - Confirm all piping joints checked for tightness after transport. |  |  |  |
|  | - Confirm rust inhibitor thoroughly cleaned from surfaces between shaft & motor. |  |  |  |
|  | - Confirm motor installed on shaft as per manufacturers instructions/ specifications. |  |  |  |
|  | - Confirm torque arm secured correctly to motor, attachment arm & reaction point. |  |  |  |
|  | - Confirm all hydraulic units and lines have been flushed, pigged and purged. |  |  |  |
|  | - Confirm motor hydraulic lines connected to give the correct rotation. |  |  |  |
|  | - Confirm motor has been filled with hydraulic fluid. |  |  |  |
|  | - Confirm drive pinion gear aligned with spider ring gear. |  |  |  |
|  | Confirm hydraulic power pack skid installed & secured on Belt Reeler station- all hold down bolts tensioned. |  |  |  |
|  | Confirm hydraulic reservoir filled with correct grade of oil, to the correct level. Grade : ______________ Qty : ________________ |  |  |  |
|  | Confirm all hydraulic hoses not stressed or twisted during installation. |  |  |  |
|  | Confirm all hydraulic connections & couplings are tight. |  |  |  |
|  | Confirm cooler fan rotates freely by hand. |  |  |  |
|  | Confirm that all rotating components are visually checked and are free from obstruction. |  |  |  |
|  | Confirm main spider bearing caps are closed & locking pins installed. |  |  |  |
|  | Confirm Dead Shaft Turnover Rollers bearings have been greased: |  |  |  |
|  | Grease Type: |  |  |  |
|  | Confirm spider gear & drive pinion have been lubricated. |  |  |  |
|  | Confirm main spider bearings have been greased: |  |  |  |
|  | Grease Type: |  |  |  |
|  | Confirm counterweight box and belt reeler has been inspected and all lifting lugs have been marked with correct WLL. |  |  |  |
|  | Confirm all surface protection has been completed as per SS-P118 including all touch up painting. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm all guarding is in place. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |