| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and confirm they are free from damage. |  |  |  |
|  | Confirm all tips are secure. |  |  |  |
|  | Confirm scraper(s) are correctly positioned in accordance with the design drawings and manufacturer’s recommendation and MA-0000-M-1008. |  |  |  |
|  | Confirm the unit is set up at correct angle to the belt. |  |  |  |
|  | Confirm scraper centreline is in alignment with pulley centreline. |  |  |  |
|  | Confirm all transport protectors removed from tips. |  |  |  |
|  | Confirm tips are contacting evenly across the full width of the belt. |  |  |  |
|  | Confirm unit is adequately supported. |  |  |  |
|  | Confirm all bolted connections are of the correct length, grade and tensioned correctly. |  |  |  |
|  | Primary : Confirm tension springs installed and tensioned as per vendor specifications: Breakaway contact force: ____________ kg Once checked, scrapers are to be backed off and locked away from the belt in a safe manner for No Load commissioning |  |  |  |
|  | Secondary : Confirm compression springs installed and compressed as per vendor specifications Spring compressed length: ________________ mm Once checked, scrapers are to be backed off and locked away from the belt in a safe manner for No Load commissioning |  |  |  |
|  | Tertiary : Confirm scraper can be tensioned into belt by 1 turn of adjusting screws or as per vendor specifications. Once checked, scrapers are to be backed off and locked away from the belt in a safe manner for No Load commissioning |  |  |  |
|  | Confirm jacking screws adjusted, secured and Denso protected. |  |  |  |
|  | Confirm seals are installed correctly at chute penetrations. |  |  |  |
|  | Confirm all guards are in place and secure. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |