| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and confirm they are free from damage. |  |  |  |
|  | Confirm installation is checked against Design Drawings and Manufacturers Specifications. |  |  |  |
|  | Confirm that Vibration Mounts are not bridged. |  |  |  |
|  | Confirm all temporary transport Shaft Locks and Bracing removed as per IOM. |  |  |  |
|  | Confirm all bolted connections are of the correct length, grade and tensioned correctly. |  |  |  |
|  | Confirm all Inlet and Outlet Ducting is correctly installed and orientated. |  |  |  |
|  | Inspect Inlet Filters and confirm correct fit and seal. |  |  |  |
|  | Confirm bearings have been greased. Grease Type: _______ |  |  |  |
|  | Confirm labyrinth seals purged. Grease Type: ______ |  |  |  |
|  | Confirm grease cup/autolube has been installed. Grease Type: ______ |  |  |  |
|  | Confirm remote grease lines installed where points are not accessible complete with labels. |  |  |  |
|  | Confirm oil cavity/ reservoir filled & level checked. Oil Type: ______ |  |  |  |
|  | Confirm Blower housing and Frame are free of load from external connections. |  |  |  |
|  | Confirm all guarding and sound dampening is in place and adequate. |  |  |  |
|  | Rotate Blower by hand to ensure there is no binding of blower or drive motor. |  |  |  |
|  | Confirm all inspection covers installed and tight. |  |  |  |
|  | Confirm Vendor ITR completed and attached (if required). |  |  |  |
|  | Confirm all guards are in place |  |  |  |
|  | Confirm all signage & equipment labelling is correct & complete as per project specifications and drawings. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |