| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| EQUIPMENT INFORMATION |  |  |  |
| --- | --- | --- | --- |
| CALIPER |  | POWER PACK |  |
| Manufacturer |  | Manufacturer |  |
| Model |  | Model |  |
| Serial Number |  | Serial Number |  |
|  |  | Relief Valve Setting |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and ensure they are clean and confirm free from damage. |  |  |  |
|  | Confirm installation is in accordance with the design drawings and manufacturer’s instructions. |  |  |  |
|  | Confirm brake disc is thoroughly clean and free from any traces of oil or anti corrosion fluid. |  |  |  |
|  | Confirm disc mounted to fixed bearing side of pulley and checked for run out (record results in table at bottom of sheet). |  |  |  |
|  | Confirm disc-grade bolts, bolt length, nuts and washers installed as per OEM specifications. |  |  |  |
|  | Confirm disc bolts tensioned in accordance with manufacturer’s recommendation & torque check sheet attached with calibration certificates and torque chart (Bar, PSI, Pa, kPa or other). Circle as required: - Torque head type: - Bolt torque value (Nm): - Calibration certificate attached: |  |  |  |
|  | Confirm Denso tape has been applied to bolt head and nuts. |  |  |  |
|  | Confirm brake calliper has been installed and aligned as per manufacturer’s instructions. |  |  |  |
|  | Confirm brake calliper hold down bolts are of the correct length, grade and tensioned as per OEM specification (attach calibration cert). Bolt torque value - Nm: |  |  |  |
|  | Confirm brake pads are clean and free from oil and pad retraction screws, springs and retaining blocks installed. |  |  |  |
|  | Confirm air gap between brake pad and disc. Gap :_______________mm |  |  |  |
|  | Confirm the hydraulic circuit is correct as per vendor drawings. |  |  |  |
|  | Confirm all hose/ pipe joints inspected & fittings are tight. |  |  |  |
|  | Confirm hydraulic lines adequately supported & clamped. |  |  |  |
|  | Confirm all flexible lubrication hoses not stressed or twisted during installation & have unrestricted movement. |  |  |  |
|  | Confirm all hydraulic hoses adequately protected at any possible chafing point. |  |  |  |
|  | Confirm all hoses have a flexible sheath installed over them to prevent damage or injury in the event of a leak. |  |  |  |
|  | Confirm all hydraulic lines flushed or pigged before final connection. |  |  |  |
|  | Confirm the hydraulic unit has been flushed and filled with the correct quantity and grade of oil. Grade :____________________ Qty : ______________________ |  |  |  |
|  | Confirm the hydraulic accumulator has been inspected and ITR M1.001 has been completed. |  |  |  |
|  | Confirm hydraulic cabinet installed as per design and cabinet functions as intended. |  |  |  |
|  | Confirm filter elements installed. |  |  |  |
|  | Confirm reservoir breather installed. |  |  |  |
|  | Confirm guarding installed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm Vendor ITR completed and attached to M1.099. |  |  |  |

| DISC RUN OUT |  |
| --- | --- |
| DISC FACE AXIAL (MEASURED AT DISC PERIPHERY) | mm |
| RADIAL | mm |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |