| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

- If units are vendor supplied fully coupled & aligned, mark items as “Vendor” & attach appropriate Vendor Factory Test Sheets and any on site confirmation.

| Item | Description |  |
| --- | --- | --- |
|  | Coupling Location |  |
|  | Manufacturer |  |
|  | Type |  |
|  | Size |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm all components are free from damage. |  |  |  |
|  | Confirm coupling and coupling components have been assembled in accordance with manufacturer’s specification. |  |  |  |
|  | Confirm flange hubs secured to shaft as per manufacturer’s specification. |  |  |  |
|  | Confirm coupling free from all foreign material and all mating surfaces cleaned. |  |  |  |
|  | Confirm opposing coupling faces have been matched low to high spots. |  |  |  |
|  | Confirm all bolted connections are of the correct length, grade and tensioned correctly and nuts and washers installed as per OEM specifications. |  |  |  |
|  | Confirm bolts are tensioned in accordance with manufacturer’s recommendation & torque check sheet attached with calibration certificates and torque chart (Bar, PSI, Pa, kPa, other). Circle as required: - Torque head type: - Bolt torque value (Nm): - Bolt lubrication (circle as required): - Dry: - Machine oil: - Other: |  |  |  |
|  | Confirm coupling has been aligned to meet the requirements of SS-M118 and any vendor specifications. Where the manufacturer and RTIO tolerances are contradictory, clarification shall be requested from the Engineer. Alignment report to be witnessed, approved and attached to this document. |  |  |  |
|  | Confirm Denso tape has been applied to bolt head and nuts as required. |  |  |  |
|  | Confirm adequate guarding securely fitted and clear of all rotating elements. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |