| Tag No: «Name» - «Description» | Task No. «TaskName» |
| --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |
| Drawing No. «DocumentList» |  |
| Additional Information: «Comments» |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm inlet & outlet of chute are free from any debris. |  |  |  |
|  | Confirm hydraulic hoses not stressed or twisted during full range of motion and pressurisation. |  |  |  |
|  | Confirm all guarding clear of any moving parts through full motion of travel. |  |  |  |
|  | Confirm limit switches/ proximity switches set correctly and functioning. |  |  |  |
|  | Confirm rams operate smoothly during full stroke in both directions. |  |  |  |
|  | Confirm gate travels smoothly during full travel in both direction. |  |  |  |
|  | Record time for gate to open | sec |  |  |
|  | Record time for gate to close | sec |  |  |
|  | Confirm gate locking pins can be installed and removed without excessive force. |  |  |  |
|  | Confirm hydraulic system level correct through full range of motion. |  |  |  |
|  | Confirm no leaks observed from hydraulic system. |  |  |  |
|  | Confirm no damage observed on ram dust seals. |  |  |  |
|  | Confirm any Vendor ITR have been. (note in appendices table) |  |  |  |
|  | Confirm the Gate is ready for Load Commissioning. |  |  |  |

| Appendices (vibration analysis, thermal imaging, vendor ITR etc…) |  |  |
| --- | --- | --- |
| Item | Vendor Document Name / Type | Appended to ITR (Initial) |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |