| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| NAME PLATE DATA | Gearbox / Reducer |
| --- | --- |
| Manufacturer |  |
| Model |  |
| Serial No |  |
| Ratio |  |
| Output rpm |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect and confirm the gearbox has no damage. |  |  |  |
|  | Confirm all components have been checked for installation completion. |  |  |  |
|  | Confirm installation is as per the manufacturer’s instructions. |  |  |  |
|  | Confirm all flushing of the gearbox and lubrication unit has been completed. |  |  |  |
|  | Confirm all lubrication first fills have been carried out to vendor specifications and samples taken and results attached. |  |  |  |
|  | Confirm direction of rotation physically checked and correct as per OEM labels. |  |  |  |
|  | Confirm direction of rotation marked on gearbox. |  |  |  |
|  | Holdback direction confirmed as correct & high speed shaft can be rotated in required direction. |  |  |  |
|  | Confirm torque arm/pad install is as per OEM specification. |  |  |  |
|  | Confirm all hold down bolts tensioned correctly. |  |  |  |
|  | Confirm no Soft Foot is evident. |  |  |  |
|  | Confirm Sun cover installed. (if required) |  |  |  |
|  | Confirm breathers correctly installed and protective seals removed. |  |  |  |
|  | Confirm condition monitoring equipment installed as per design requirements. |  |  |  |
|  | Confirm drain valve closed and plug installed. |  |  |  |
|  | Confirm bund drain point installed & valve is in closed position complete with plug. |  |  |  |
|  | Confirm all packaging, debris removed from drive and sump cleaned including any hydrocarbon spills. |  |  |  |
|  | Confirm all grease points are accessible or have remote lines reporting to a manifold and labelled as per design specification. |  |  |  |
|  | Manually rotate gearbox input shaft whilst monitoring output shaft rotation to confirm gearbox ratio is correct. Input Shaft number of rotations _____to 1 rotations of output shaft. (if achievable) |  |  |  |
|  | Confirm surface protection is as per SS-P118 and all touch up painting has been completed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |