| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Manufacturer |  |
| --- | --- |
| Design Reg. Number |  |
| Hoist Manufacturer |  |
| Hoist Model |  |
| Hoist Capacity |  |
| Span | m |
| Crane rail size | kg/m |

| Item | Description | OK |  | N/A | Punch |
| --- | --- | --- | --- | --- | --- |
|  | Inspect all components and confirm they are free from damage. |  |  |  |  |
|  | Confirm installation against design drawings and vendor data. |  |  |  |  |
|  | Confirm all bolted connections have been checked for tension and structural torque sheets have been completed. |  |  |  |  |
|  | Confirm correct tag number is shown on trolley. |  |  |  |  |
|  | Confirm correct tag number is shown on chain block. |  |  |  |  |
|  | Confirm WLL of trolley and chain block match monorail and design drawings. |  |  |  |  |
|  | Confirm chain block operates freely and has been inspected, tagged and in date. |  |  |  |  |
|  | Confirm hook has spring return safety catch and in good working order. |  |  |  |  |
|  | Confirm trolley moves along entire monorail without binding or excessive force. |  |  |  |  |
|  | Confirm chain is accessible location for operators and does not present operational hazards. |  |  |  |  |
|  | Confirm full travel zone has been checked and is clear of any obstructions in the path of travel. |  |  |  |  |
|  | Confirm maintenance access platform is as per design & suited to requirements and gates self-close and locked. |  |  |  |  |
|  | Confirm guarding is installed correctly & secure. |  |  |  |  |
|  | Confirm WLL Painted on crane beams as per MA-0000-S-1005 and AS Standards. |  |  |  |  |
|  | Confirm Classified Plant identification of data plate, design registration number and equipment number is marked on the machine as per EP-A003 Classified Plant Management Procedure. |  |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |  |
|  | Confirm all rails have been thoroughly inspected for debris prior to being put into service. |  |  |  |  |
|  | Confirm all painting and surface protection has been completed as per SS-P118 and all touch up painting has been completed. |  |  |  |  |
|  | Confirm verified and approved as-built ‘Red line” drawings are attached, witnessed by the engineer and approved for official transmittal. |  |  |  |  |
|  | Confirm nameplate photo attached. |  |  |  |  |
|  | Confirm all outstanding items have been listed with photo evidence, punch list number assigned and recorded. |  |  |  |  |
|  | Confirm all OEM inspection check sheets completed and attached. (M1.099) |  |  |  |  |
|  | Confirm crane structural document (S1.006) has been completed, witnessed and approved. |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |