| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm Truss/ Module/ Shuttle etc. Number: (attach mark- up drawing for reference complete with survey report). |  |  |  |
|  | Confirm all frames and idler types installed in correct locations as per Profile and Data drawings. |  |  |  |
|  | Confirm transition idler frames installed as per design requirements. |  |  |  |
|  | Confirm carry and return idler alignment completed & survey report attached with marked up section on Profile and Data. |  |  |  |
|  | Confirm packers installed under idlers as stated in design drawings. |  |  |  |
|  | Confirm packers installed under conveyor modules and survey report attached. |  |  |  |
|  | Confirm all transition idler wing rollers adjusted correctly and frames trimmed to below belt level. |  |  |  |
|  | Confirm all bolts tensioned on idler support frames & oversize, hardened washers are installed. |  |  |  |
|  | Confirm all rollers are seated full depth in the slot in the support frame. |  |  |  |
|  | Confirm non-magnetic idlers used in proximity of metal detector. |  |  |  |
|  | Confirm all stone guards are fitted where specified and not catching. |  |  |  |
|  | Confirm all breather holes are at the lower end for all inclined (wing) rollers. |  |  |  |
|  | Confirm idler retaining clips installed in tramp metal magnet zone if steel shell idlers used. |  |  |  |
|  | Confirm idler retaining clips installed as required on suspended frames. |  |  |  |
|  | Confirm all wind guards fitted where specified. |  |  |  |
|  | Confirm idlers have ±2 mm of adjustment in mounting slots. |  |  |  |
|  | Confirm all training idlers are installed correctly in relation to belt direction and locked square to conveyor centre line. |  |  |  |
|  | Confirm drop down idlers fixed in correct position & pins installed complete with safety chain. |  |  |  |
|  | Confirm carry idler frames numbered in accordance with design drawings and 3D Model. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm all guarding is installed and secure. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |