| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm materials of construction are as per specifications and design drawings. |  |  |  |
|  | Confirm support structure complete, level and accepted. |  |  |  |
|  | Confirm construction and installation is as per MA-0000-M-1103, MA-0000-M-1104, MA-0000-M-1105, MA-0000-M-1106 and MA-0000-M-1109. |  |  |  |
|  | Confirm all welding works completed and tested as required. |  |  |  |
|  | Confirm all bolted connections and fixing systems including hold down, flange and liner are tensioned and marked correctly. |  |  |  |
|  | Confirm all supporting structures are secure and all bolts fitted and tensioned. |  |  |  |
|  | Confirm protective coatings applied to suit SS-P118 and RVS-0000-SPE-S-003 and all touch-up painting completed. |  |  |  |
|  | Confirm impact plate is aligned with equipment receiving feed, in accordance with standards and specifications and survey(s) attached. |  |  |  |
|  | Confirm impact plate has been set up in accordance with the design drawings. |  |  |  |
|  | Confirm impact plate spray bars installed as per drawing or blanking covers installed if N/A. |  |  |  |
|  | Confirm liners and required packers are installed as per design drawing and all bolted connections are of the correct length, grade and tensioned correctly. Joint gaps as per standard drawing MA-0000-M-1114. |  |  |  |
|  | Confirm impact plate moves through full range of motion without contacting chute, pulley, cutter etc… |  |  |  |
|  | Confirm turn buckle and slide bar move freely through full range of motion, once confirmed, turn buckle and all slide bar nuts have been tightened, turn buckle and bolts/nuts have been denso taped and impact plate has been left in correct location as per design. |  |  |  |
|  | Confirm bearings/pivot brackets have been installed correctly, greased and locked in place as required. |  |  |  |
|  | Confirm signage & equipment labelling installed as per design drawings and specifications. |  |  |  |
|  | Confirm all scaffold, debris and waste material removed and remedial surface treatment completed as per specification. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |