| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm materials of construction are as per specifications and drawings. |  |  |  |
|  | Confirm support structure complete, level and accepted. |  |  |  |
|  | Confirm all welding works completed and tested as required. (non rubber lined items only) |  |  |  |
|  | Confirm all bolted connections and fixing systems including hold down, flange and liner are tensioned and marked correctly. |  |  |  |
|  | Confirm all supporting structure and covers are secure and all bolts tensioned/ wedges fitted. |  |  |  |
|  | Confirm all inspection doors and openings, including seal arrangements are correctly fabricated and installed in accordance with the drawings and locking mechanism freely operate and both doors close & seal correctly. |  |  |  |
|  | Confirm protective coatings applied to suit SS-P118 and all touch-up painting completed. |  |  |  |
|  | Confirm feed chutes are aligned with equipment receiving feed, in accordance with standards and specifications and survey(s) attached. |  |  |  |
|  | Confirm all impact plates and wear items have been installed and set up in accordance with the design drawings. |  |  |  |
|  | Confirm all protective rubber liners have been installed, no damage is evident and chutes are marked “NO WELDING – RUBBER LINED” as per SS-M158. (where required) |  |  |  |
|  | Confirm spray bars installed as per drawing. |  |  |  |
|  | Confirm blocked chute devices and level devices installed as per design drawings. |  |  |  |
|  | Confirm all feed spouts and chutes are installed as per design drawings. |  |  |  |
|  | Confirm signage & equipment labelling is installed as per design drawings and specifications. |  |  |  |
|  | Confirm all scaffold, debris and waste material removed and remedial surface treatment completed as per specification. |  |  |  |

TYPICAL SURVEY TEMPLATE

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |