| Tag No: «Name» - «Description» | Task No. «TaskName» |
| --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |
| Drawing No. «DocumentList» |  |
| Additional Information: «Comments» |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm grease pump motor tested & correct direction of rotation achieved: |  |  |  |
|  | Grease pump motor: | CW/CCW |  |  |
|  | Confirm grease pot filled and sealed (if required): |  |  |  |
|  | Grease type: |  |  |  |
|  | Confirm all grease lines checked for leaks and weeps at compression fittings. |  |  |  |
|  | Confirm grease manifold block and solenoid valves function correctly. |  |  |  |
|  | Confirm unit operation interval is correct. |  |  |  |
|  | With the lubrication feed points disconnected and bagged, confirm the grease delivery volume at each lubrication point is correct as per design. |  |  |  |
|  | Confirm the grease delivery pressure is correct as per design. (delivery pressure will be reduced as the system is open ended) |  |  |  |

HYDRAULIC LUBRICATION PACK & COOLER

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm all motors bump tested & correct direction of rotation achieved: (viewed from non drive end) |  |  |  |
|  | Hydraulic Pump Motor 1: | CW/CCW |  |  |
|  | Hydraulic Pump Motor 2: | CW/CCW |  |  |
|  | Oil Cooler Pump Motor: | CW/CCW |  |  |
|  | Oil Cooler Fan Motor: | CW/CCW |  |  |
|  | Confirm motor/ pump couplings installed & aligned after direction test (if required). |  |  |  |
|  | Confirm motor bearings greased (if required): |  |  |  |
|  | Grease type: |  |  |  |
|  | Confirm pump bearings greased (if required): |  |  |  |
|  | Grease type: |  |  |  |
|  | Confirm full system flush conducted, results attached & looped lines re- connected. |  |  |  |
|  | Carry out an unloaded run of the system for minimum 4 hours or until bearing temperatures stabilise and record temperatures in the tables below. |  |  |  |
|  | Confirm filter elements & filter bowls checked for contaminants & changed out if required. |  |  |  |
|  | Confirm all hydraulic fittings, hoses and couplings checked for leaks. |  |  |  |
|  | Confirm tank level after system fully charged & refill as per manufacturer’s specifications. |  |  |  |
|  | Confirm pump discharge pressure and flow corresponds to specifications. |  |  |  |
|  | Confirm representative oil sample taken and oil analysis report attached. |  |  |  |
|  | Confirm all Vendor ITR and supporting documents have been completed. (note in appendices table |  |  |  |

| Appendices (vibration analysis, thermal imaging, vendor ITR etc…) |  |  |
| --- | --- | --- |
| Item | Vendor Document Name / Type | Appended to ITR (Initial) |

| TEMPERATURE & PRESSURE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Time (mins) | Main Tank Oil Temp | Cooling Unit Temp In | Cooling Unit Temp Out | Pump Charge Pressure | Ambient Temp °C |
| 15 | °C | °C | °C |  | °C |
| 30 | °C | °C | °C |  | °C |
| 60 | °C | °C | °C |  | °C |
| 90 | °C | °C | °C |  | °C |
| 120 | °C | °C | °C |  | °C |
| 180 | °C | °C | °C |  | °C |
| 240 | °C | °C | °C |  | °C |

| Time (mins) | BRG 1 | BRG 2 | BRG 3 | BRG 4 | BRG 5 | BRG 6 | Ambient Temp °C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | °C | °C | °C | °C | °C | °C | °C |
| 30 | °C | °C | °C | °C | °C | °C | °C |
| 60 | °C | °C | °C | °C | °C | °C | °C |
| 90 | °C | °C | °C | °C | °C | °C | °C |
| 120 | °C | °C | °C | °C | °C | °C | °C |
| 180 | °C | °C | °C | °C | °C | °C | °C |
| 240 | °C | °C | °C | °C | °C | °C | °C |

Record bearing locations in the table below.

| Bearing No. | Location Description |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |