| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

If any items are vendor supplied, sealed or calibrated units, then please mark items as “Vendor” & attach appropriate Factory Test Sheets. If vendor verification can’t be supplied, then item checks are required to be undertaken.

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| MAGNET |  |  |  |  |
|  | Inspect all components and confirm they are free of damage. |  |  |  |
|  | Confirm installation and materials of construction are as per specification and drawings. |  |  |  |
|  | Whilst magnet is cold & horizontal, confirm oil level through plug on side of magnet. Complete first fills if required. |  |  |  |
|  | Confirm magnet has been inspected and no oil leaks are evident. |  |  |  |
|  | Confirm all lifting lugs are stamped with the correct WLL. |  |  |  |
|  | Confirm magnet is slung from the 4 outer lugs as per the drawings. |  |  |  |
|  | Confirm slings/ chains & shackles/ hammerlocks have the correct WLL and safety wired. |  |  |  |
|  | Confirm correct WLL is stencilled on both sides of magnet. |  |  |  |
|  | Confirm position of pressure relief valve is at the highest point of the magnet. |  |  |  |
|  | Confirm the pressure relief valve (breather) is free of dirt and obstructions. |  |  |  |
|  | Confirm magnet is aligned parallel and square to the belt line. |  |  |  |
|  | Confirm distance from underside of magnet to top of belt is per manufacturer’s specification. |  |  |  |
|  | Confirm magnet incline is as per Profile and Data and OEM design specification. |  |  |  |
|  | Confirm discharge chute is correctly aligned to receive tramp metal. |  |  |  |
|  | Confirm that warning signs erected in appropriate areas- heat/ hearing/ magnetic field pacemaker as per design drawings and project specifications. |  |  |  |
|  | Confirm that appropriate guarding is in place. |  |  |  |
|  | Confirm conveyor idlers have retaining clips installed or use of non-magnetic idlers. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm Vendor ITR M1.099 has been completed, witnessed and approved. |  |  |  |
| TROLLEY & RAILS |  |  |  |  |
|  | Confirm monorail supplied/fabricated correct as per design drawings and conforms to MA-0000-S-1005 and MA-0000-M-1064. |  |  |  |
|  | Confirm trolley rails level & square (attach survey report). |  |  |  |
|  | Confirm all bolted connections are of the correct length, grade and tensioned correctly. |  |  |  |
|  | Confirm WLL displayed in accordance with MA-0000-S-1005. |  |  |  |
|  | Confirm all trolley wheels contacting rails. |  |  |  |
|  | Confirm all bolted connections disassembled for the installation of the trolley onto the rails have been re-tensioned correctly. |  |  |  |
|  | Confirm trolley ring gears have been greased. |  |  |  |
|  | Confirm trolley drive shaft pillow blocks have been greased. |  |  |  |
|  | Confirm drive gearbox oil level checked. |  |  |  |
|  | Confirm rail end stops installed. |  |  |  |
|  | Confirm catenary rail & supports have been installed as per drawings and are clear of any obstructions in the travel zone. |  |  |  |
|  | Confirm all debris have been removed from the track. |  |  |  |
|  | Confirm Classified Plant registration number, data plate and equipment number are clearly displayed in accordance with EP-A003. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm Vendor ITR (M1.099) has been completed, witnessed and approved. |  |  |  |
|  | Confirm Contractor ITR (M1.098) has been completed, witnessed and approved. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |