| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. | «OEMPartNumber» |  |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. |  | «TaskName» |
| Additional Information: | «Comments» |  |  |  |

| - GENERATOR |  | - MOTOR |  |
| --- | --- | --- | --- |
| Manufacturer |  | Manufacturer |  |
| Model |  | Model |  |
| Serial Number |  | Serial Number |  |

- FUEL / LUBRICATION / COOLANT

| EQUIPMENT FILL LIST | MANUFACTURER | TYPE/ GRADE | QUANTITY |
| --- | --- | --- | --- |
|  |  |  | L |
|  |  |  | L |
|  |  |  | L |
|  |  |  | L |
|  |  |  | L |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm installation is in accordance with the drawings & manufacturer’s instructions. |  |  |  |
|  | Confirm a thorough visual inspection undertaken to check for any transport damage. |  |  |  |
|  | Confirm all bolts tensioned correctly. |  |  |  |
|  | Confirm all drive belts are aligned and tensioned correctly. |  |  |  |
|  | Confirm all first fills (fuel/oil/water/coolant) have been completed, recorded and samples taken. |  |  |  |
|  | Confirm all pipe work, hosing flushed out, clear of debris, treated & correctly installed. |  |  |  |
|  | Confirm all pipe work connected as per P&ID’s, design drawings and manufacturer’s instructions. |  |  |  |
|  | Confirm all pipe work is adequately supported. |  |  |  |
|  | Confirm all breathers fitted. |  |  |  |
|  | Confirm all filters fitted. |  |  |  |
|  | Confirm all doors & access panels shut & secured. |  |  |  |
|  | Confirm all moving parts clear of all structure, guards & other equipment. |  |  |  |
|  | Confirm all rotating elements can be freely rotated by hand. |  |  |  |
|  | Confirm all safety equipment & protection devices installed in accordance with the manufacturer’s instructions. |  |  |  |
|  | Confirm all debris & waste material removed from area. |  |  |  |
|  | Confirm Vendor ITR (M1.099) has been completed, reviewed, approved and attached. |  |  |  |
|  | Confirm all signage & equipment labelling correct & complete as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |