| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MOTOR |  | POWER PACK |  |  |  |
| Manufacturer |  | Manufacturer |  |  |  |
| Model |  | Model |  |  |  |
| Serial Number |  | Serial Number |  |  |  |
| COOLING UNIT |  | HYDRAULIC FLUID |  |  |  |
| Manufacturer |  | Manufacturer |  |  |  |
| Model |  | Grade |  |  |  |
| Serial Number |  | Tank | L | Motor | L |

HYDRAULIC POWER PACK & COOLER

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Check all components and confirm they are free from damage. |  |  |  |
|  | Confirm installation is in accordance with the manufacturer’s instructions and design drawings. |  |  |  |
|  | Confirm power pack cabinet located to allow specified clearance on all sides. |  |  |  |
|  | Confirm power pack & cooling unit hold down bolts are tensioned correctly. |  |  |  |
|  | Confirm hydraulic fittings/connections correct as per design and project specifications. |  |  |  |
|  | Confirm all hydraulic connections tightened. |  |  |  |
|  | Confirm all hydraulic hoses not stressed or twisted during installation and have spiral wound protection where penetrating trough structure/ flooring. |  |  |  |
|  | Confirm hoses and pipework supported and secured/restrained/clamp appropriately. |  |  |  |
|  | Confirm hydraulic lines and tanks are cleaned, flushed and pigged as per RTIO and vendor requirements and flushing report is attached. |  |  |  |
|  | Confirm motor drain lines connected to hydraulic tank with minimum restrictions. |  |  |  |
|  | Confirm cooling unit oil lines connected & fans rotate freely by hand. |  |  |  |
|  | Confirm filter elements installed, flow direction correct & inspected before first fill. |  |  |  |
|  | Confirm filters installed with correct micron rating. |  |  |  |
|  | Confirm reservoir breather installed. |  |  |  |
|  | Confirm hydraulic tank filled to level marker on site gauge. |  |  |  |
|  | Confirm uppermost vent plug removed from pump & pre- filled with oil via drain plug until all air is purged. |  |  |  |
|  | Confirm bund drain line with valve in closed position complete with plug. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm stainless steel schematic plaque located next to unit. |  |  |  |

HYDRAULIC MOTOR

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Check all components and confirm they are free from damage. |  |  |  |
|  | Confirm motor installed on shaft as per manufacturer’s instructions. |  |  |  |
|  | Confirm motor mounting shaft thoroughly cleaned prior to mounting motor. |  |  |  |
|  | Confirm motor rotation is such that the two drain ports are positioned vertically. |  |  |  |
|  | Confirm torque arm fitted to motor & bolts tensioned correctly: |  |  |  |
|  | Torque: |  |  |  |
|  | Confirm torque arm correctly secured to attachment arms & reaction point. |  |  |  |
|  | Confirm motor hydraulic lines connected to give correct direction of rotation. |  |  |  |
|  | Confirm all hydraulic connections and couplings are tight. |  |  |  |
|  | Confirm all cavities which require initial fills are filled with oil, plugs re-tensioned & no oil leaks are evident. |  |  |  |
|  | Confirm motor filled through uppermost plug in outer front casing with a minimum of 10 micron filtered oil or as per manufacturer’s specification. |  |  |  |
|  | Confirm motor drain line connected. |  |  |  |
|  | Confirm all guarding installed & secure. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |