| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| - PUMP |  | - MOTOR |  | - TRAILER |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | - Manufacturer |  | - Manufacturer |  |
| Model |  | - Model |  | - Model |  |
| Serial Number |  | - Serial Number |  | - Serial Number |  |

| PUMP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | OK | N/A | Punch |
|  | Confirm all components have been inspected and are free from damage. |  |  |  |
|  | Confirm impeller shaft and motor are free to rotate and have been rotated by hand. |  |  |  |
|  | Confirm all hold down bolts are correctly tensioned and no soft foot evident. |  |  |  |
|  | Confirm all suction and discharge piping, valves, gauges and fittings have been installed in accordance with the drawings. |  |  |  |
|  | Confirm all flange bolting checked for correct tension. |  |  |  |
|  | Confirm pump direction of rotation clearly marked. |  |  |  |
|  | Confirm pump bearings have been greased. Grease Type: _______________ |  |  |  |
|  | Confirm labyrinth seals purged. Grease Type: ______________ |  |  |  |
|  | Confirm grease cup/autolube installed. Grease Type: ______________ |  |  |  |
|  | Confirm oil cavity/ reservoir filled & level checked. Oil Type: ______________ |  |  |  |
|  | Confirm mechanical seal has been set correctly and any transport locks have been removed. |  |  |  |
|  | Confirm “Run Dry” oil reservoir has been filled and level checked. Oil Type_______________ |  |  |  |
|  | Confirm seal/gland water plumbed in. |  |  |  |
|  | Confirm all self prime connections are connected and tight. |  |  |  |
|  | Confirm drive alignment has been completed and approved and alignment report is attached. |  |  |  |
|  | Confirm all guarding is installed and secure. |  |  |  |
|  | Confirm all identification and signage installed as per design and drawings and project specification. |  |  |  |

| MOTOR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | OK | N/A | Punch |
|  | Confirm all components have been inspected and are free from damage. |  |  |  |
|  | Confirm oil level has been checked and meets manufacturer’s specification. |  |  |  |
|  | Confirm coolant level is correct as per manufacturer’s specification. |  |  |  |
|  | Confirm all drive belts are aligned and tensioned correctly. |  |  |  |
|  | Confirm all hoses, filters, fixtures and fittings are installed and connections are tight. |  |  |  |
|  | Confirm all guarding is installed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| TRAILER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Description | OK | N/A | Punch |
|  | Confirm all components have been inspected and are free from damage. |  |  |  |
|  | Confirm the pump trailer is installed in accordance with the manufacturer’s installation instructions. |  |  |  |
|  | Confirm 4 off wheel chocks are affixed to trailer. |  |  |  |
|  | Confirm tow hitch is affixed and tight. |  |  |  |
|  | Confirm stabiliser legs are affixed and in good condition. |  |  |  |
|  | Confirm all wheels and tyres including the spare, are free from damage and all bolted connections are tight. |  |  |  |
|  | Confirm trailer is furnished with 2 off fire extinguishers and they have been inspected, tagged and are in date. |  |  |  |
|  | Confirm suction boom and mechanical winch are free of damage. |  |  |  |
|  | Confirm suction boom is painted Yellow in accordance with RTIO and Australian standards. |  |  |  |
|  | Confirm the suction boom and mechanical winch have been clearly marked with the WLL and all classified plant inspections have been carried out as required. |  |  |  |
|  | Confirm trailer surface protection meets SS-P118. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |