| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

JAW CRUSHER

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and ensure they are free from damage. |  |  |  |
|  | Confirm all transport protection has been removed. |  |  |  |
|  | Confirm crusher is installed as per manufacturer’s specifications. |  |  |  |
|  | Confirm correct sized/ grade rubber dampers installed and are not bridged. |  |  |  |
|  | Confirm crusher hold down bolts tensioned correctly. |  |  |  |
|  | Confirm all cheek plate bolts tensioned correctly. |  |  |  |
|  | Confirm jaw liners seated correctly & hold down wedge bolts checked for correct tension. |  |  |  |
|  | Confirm drive alignment has been completed, witnessed and approved and alignment report is attached. |  |  |  |
|  | Confirm motor hold down bolts are of the correct length, grade and tensioned correctly. |  |  |  |
|  | Confirm no soft foot is evident on motor/base. |  |  |  |
|  | Confirm Pitman shaft bearing hold down bolts are of the correct length, grade and tensioned correctly. |  |  |  |
|  | Confirm Pitman shaft bearings greased: |  |  |  |
|  | - Brand: |  |  |  |
|  | - Type: |  |  |  |
|  | Confirm toggle plate seated correctly in both upper & lower mounts. |  |  |  |
|  | Confirm closed side setting set as per operational requirements _________________mm |  |  |  |
|  | Confirm dust seal on lower toggle seat is set correctly. |  |  |  |
|  | Confirm both feed & discharge chutes are central to jaw crusher. |  |  |  |
|  | Confirm that the crusher cavity is empty. |  |  |  |
|  | Confirm all guarding is installed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm Vendor ITR M1.099 has been completed. |  |  |  |

- CONE CRUSHER

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and ensure they are free from damage. |  |  |  |
|  | Confirm crusher installed as per OEM instructions. |  |  |  |
|  | Confirm crusher seals installed and secure. |  |  |  |
|  | Confirm all motor, crusher and support frame hold down bolts are tensioned. |  |  |  |
|  | Confirm drive alignment has been completed, witnessed and approved and alignment report is attached. |  |  |  |
|  | Confirm that the feed cone and crushing chamber are empty. |  |  |  |
|  | Confirm that the crusher revolves freely by hand. |  |  |  |
|  | - Confirm crusher discharge chute installed & aligned central to crusher. |  |  |  |
|  | Confirm oil level is correct in the sump. |  |  |  |
|  | Confirm the correct oil type and oil level in lube pump as per manufacturer’s specification: Oil Type: ________________ Oil Level:________________ |  |  |  |
|  | Confirm there are no oil leaks from around the drive shaft housing, bell seal area or lube hoses between crusher sump, oil filter & flow sensor. |  |  |  |
|  | Confirm lubrication hoses between crusher sump, oil filter and flow sensor are correct as per design and specification. |  |  |  |
|  | Confirm the guard is not in contact with belts, pulleys or shafts. |  |  |  |
|  | Confirm all lubrication points are accessible. |  |  |  |
|  | Confirm the tramp magnet is installed in crusher feed chute. |  |  |  |
|  | Confirm the dust curtain is installed correctly. |  |  |  |
|  | Confirm all guarding is installed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm Vendor ITR M1.099 has been completed. |  |  |  |

- LUBRICATION UNIT

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Inspect all components and ensure they are free from damage. |  |  |  |
|  | Confirm all transport protection removed. |  |  |  |
|  | Confirm all grease line fittings inspected and tightened. |  |  |  |
|  | Confirm correct grease type and level in lubrication unit. |  |  |  |
|  | Confirm correct oil type and oil level in lubrication unit: |  |  |  |
|  | - Brand |  |  |  |
|  | - Type |  |  |  |
|  | Confirm the lubrication unit and all associated hosing and pipework has been pigged, flushed and dried as per SS-M198. |  |  |  |
|  | Confirm Vendor ITR M1.099 has been completed. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |