| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| - EQUIPMENT |  | - MOTOR |  |
| --- | --- | --- | --- |
| Manufacturer |  | Manufacturer |  |
| Model |  | Model |  |
| Serial Number |  | Serial Number |  |

- LUBRICATION

| EQUIPMENT LIST | MANUFACTURER | TYPE/ GRADE | QUANTITY |
| --- | --- | --- | --- |
|  |  |  | L |
|  |  |  | L |
|  |  |  | L |
|  |  |  | L |
|  |  |  | L |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm installation is in accordance with the drawings & manufacturer’s instructions. |  |  |  |
|  | Confirm a thorough visual inspection has been undertaken to check for any transport damage. |  |  |  |
|  | Confirm all bolts have been tensioned correctly. |  |  |  |
|  | Confirm all pipe work is adequately supported. |  |  |  |
|  | Confirm all lubrication pipe work/hosing has been pigged, flushed, clear of debris and purged. |  |  |  |
|  | Confirm all lubrication first fills have been completed, samples taken and results attached. |  |  |  |
|  | Confirm all breathers fitted. |  |  |  |
|  | Confirm all filters fitted. |  |  |  |
|  | Confirm all guarding is installed & secured as per the design drawings. |  |  |  |
|  | Confirm all inspection access panels shut & secured. |  |  |  |
|  | Confirm tramp door, tramp door locking pins and associated ancillary items have been installed correctly as per the design drawings. |  |  |  |
|  | Confirm all moving parts clear of all structure, guards & other equipment. |  |  |  |
|  | Confirm all safety equipment & protection devices installed in accordance with the manufacturer’s instructions. |  |  |  |
|  | Confirm the inching drive has been site fitted to prove alignment. (removed after checking) |  |  |  |
|  | Confirm all travel drives and ancillary items are installed, aligned and tensioned correctly as per vendor and design drawings. (attach rail survey report) |  |  |  |
|  | Check and confirm all alignments on equipment have been completed and alignment reports witnessed and attached. |  |  |  |
|  | Confirm all equipment specific ITR have been completed and witnessed for Gearboxes, Inching Drives, Travel Drives, Fluid Couplings etc... |  |  |  |
|  | Confirm the sizer is correctly located for feed and discharge of material as per design drawings. |  |  |  |
|  | Confirm all debris & waste material have been removed from area. |  |  |  |
|  | Confirm surface protection is as per SS-P118 and all touch up painting has been completed. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
|  | Confirm Vendor ITR (M1.099) has been completed. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |