| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm all components have been inspected and are free from damage. |  |  |  |
|  | Confirm the equipment is correctly positioned in accordance with the drawings and manufacturer’s recommendation. |  |  |  |
|  | Confirm unit is adequately supported and associated structures have been accepted. |  |  |  |
|  | Confirm the plough is set up at the correct angle to the belt and the lead in is at the correct angle. |  |  |  |
|  | Confirm fixed plough set a minimum 25mm above the highest point of the relative idlers as per MA-0000-M-1138 as specified and debris can freely flow off belt with no obstructions. |  |  |  |
|  | Confirm shedder plate is installed as per MA-0000-M-1138 and 0000-M-0048. |  |  |  |
|  | Confirm a minimum gap of 10mm between the shedder plate and underside of the belt. |  |  |  |
|  | Confirm the shedder plate does not hinder access to rollers, pull wires or any other ancillary equipment. |  |  |  |
|  | Confirm all wear liners and wear plates installed correctly. |  |  |  |
|  | Confirm concrete bunker has been installed in the correct location as per the design drawings. |  |  |  |
|  | Confirm all bolted connections are of the correct length, grade and tensioned correctly. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |