| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
|  | Confirm the pulley carriage and rails are installed and aligned in accordance with the drawings & the survey report is attached. |  |  |  |
|  | Confirm take up tower installation & alignment of carriage & guides is in accordance with the drawings & survey report attached. |  |  |  |
|  | Confirm take-up mass corresponds to the specifications on the drawings & is clearly marked on the take- up: Mass : _____________________________Tonne |  |  |  |
|  | Confirm the take up winch has been installed according to the design drawings and manufacturer’s specification. |  |  |  |
|  | Confirm all winch locking mechanisms (pawl brake) operate smoothly and can be locked. |  |  |  |
|  | Confirm all solenoid switches are activated by the pawl brake handle. |  |  |  |
|  | Confirm Classified Plant identification of data plate, design registration number and equipment number is marked on the machine as per EP-A003 Classified Plant Management Procedure. |  |  |  |
|  | Confirm all bolted connections have been checked for correct grade, length and tension. |  |  |  |
|  | Confirm the unit has been visually inspected for any possible obstructions that may limit freedom of take-up movement on both take up pulley carriage rails & take up tower. |  |  |  |
|  | Confirm all sheaves have been rotated by hand to confirm freedom of rotation. |  |  |  |
|  | Confirm the correct grade, diameter and lay of wire rope used is as per design drawings. |  |  |  |
|  | Confirm take–up in correct position with belt tensioned. |  |  |  |
|  | Confirm the wire rope is installed correctly & seats within the sheave groove. |  |  |  |
|  | Confirm all remote grease lines have been flushed, pigged and purged, are secure & manifold is correctly identified and easily accessible outside guarding. Grease Type: |  |  |  |
|  | Confirm all sheaves have been greased. |  |  |  |
|  | Confirm all lubrication first fills have been completed, samples taken and results attached. |  |  |  |
|  | Confirm keeper brackets have correct clearance under rail support beams. |  |  |  |
|  | Confirm carriage wheels seated on rail correctly. |  |  |  |
|  | Confirm carriage wheels have been greased. |  |  |  |
|  | Confirm wire rope restraints securely fastened to structure. |  |  |  |
|  | Confirm counterweight lifting system installed as per drawings. |  |  |  |
|  | Confirm all rigging equipment e.g. shackles, wedge sockets etc. are rated for the applicable load & installed/ secured correctly. |  |  |  |
|  | Confirm maintenance chains securely attached in the stowed position. |  |  |  |
|  | Confirm shedder plate installed correctly & rubbers secured to plates. |  |  |  |
|  | Confirm impact buffer has been installed correctly. |  |  |  |
|  | Confirm all guarding has been installed as per the design drawings and secure. |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |