| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. | «OEMPartNumber» |  |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. |  | «TaskName» |
| Additional Information: | «Comments» |  |  |  |

If any items are vendor supplied, sealed or calibrated units, then please mark items as “Vendor” & attach appropriate Factory Test Sheets. If vendor verification can’t be supplied, then item checks are required to be undertaken. The specifications listed in this check sheet refer to a Transmin Rock Breaker. If another manufacturer is used, then refer to the relevant IOM for specifications.

| SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - BREAKER HEAD |  |  |  |  |  | - SLEW DRIVE |  |  |  |  |  |  |  |
|  |  |  |  |  |  | - MOTOR |  |  |  | - GEARBOX |  |  |  |
| Manufacturer |  |  |  |  |  | - Manu |  |  |  | - Manu |  |  |  |
| Model |  |  |  |  |  | - Spec |  |  |  | - Spec |  |  |  |
| Serial No. |  |  |  |  |  | - Serial No. |  |  |  | - Serial No. |  |  |  |
| - POWER PACK |  |  |  |  |  | - COOLING UNIT |  |  |  |  |  |  |  |
| Manufacturer |  |  |  |  |  | Manufacturer |  |  |  |  |  |  |  |
| Model |  |  |  |  |  | Model |  |  |  |  |  |  |  |
| Serial No. |  |  |  |  |  | Serial No. |  |  |  |  |  |  |  |
| - LUBE UNIT |  |  |  |  |  | - HYDRAULIC FLUID |  |  |  |  |  |  |  |
| Manufacturer |  |  |  |  |  | - Manufacturer |  |  |  |  |  |  |  |
| Model |  |  |  |  |  | - Specification |  |  |  |  |  |  |  |
| Serial No. |  |  |  |  |  | - Reservoir |  |  | - L |  | - Motor |  | - L |
| - GREASE |  |  |  |  |  | - GEARBOX OIL |  |  |  |  |  |  |  |
| - PIVOT PIN |  |  |  | - HAMMER |  |  |  |  |  |  |  |  |  |
| - Manu |  |  |  | - Manu |  | - Manufacturer |  |  |  |  |  |  |  |
| - Spec |  |  |  | - Spec |  | - Specification |  |  |  |  |  |  |  |
| - Reservoir | - L |  |  | - Reservoir | - L | - Quantity |  |  | - L |  |  |  |  |

| Item |  |  | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- | --- | --- |
| BREAKER HEAD |  |  |  |  |  |  |
|  |  | Confirm no damage caused during transport, handling or installation. |  |  |  |  |
|  |  | Confirm rock breaker installed as per design and manufacturer’s specifications and installation drawings. Attach all vendor check sheets during the construction installation phase. |  |  |  |  |
|  |  | Confirm all boom pin retainers, nuts, bolts etc. were checked for tightness. |  |  |  |  |
|  |  | Confirm slew base orientation is correct. |  |  |  |  |
|  |  | Confirm main hold down bolts tensioned & appropriate bolt tensioning check sheet and calibration certificates attached. |  |  |  |  |
|  |  | Confirm base has been installed level. |  |  |  |  |
|  |  | Confirm hammer installed & mounting bolts tensioned. |  |  |  |  |
|  |  | Confirm breaker head park up location installed and location/structure accepted. |  |  |  |  |
|  |  | Confirm all guarding installed & secure. |  |  |  |  |
|  |  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |  |
| SLEW DRIVE |  |  |  |  |  |  |
|  |  |  | Confirm all gearbox mounting bolts were checked for correct tension after transport. |  |  |  |
|  |  |  | Confirm, set and record backlash between slew and pinion teeth. ______________mm |  |  |  |
|  |  |  | Confirm pinion gear is coated with grease as per OEM recommendations. |  |  |  |
|  |  |  | Confirm all hydraulic connections tightened. |  |  |  |
|  |  |  | Confirm gearbox filled with oil. |  |  |  |
|  |  |  | Confirm motor pre filled with hydraulic fluid. |  |  |  |
|  |  |  | Confirm motor hydraulic lines connected to give correct direction of rotation. |  |  |  |
| GREASE UNIT |  |  |  |  |  |  |
|  |  | Confirm all transport protection removed. |  |  |  |  |
|  |  | Confirm thorough inspection undertaken to identify any transport damage. |  |  |  |  |
|  |  | Confirm unit checked for any loose connections after transport. |  |  |  |  |
|  |  | Confirm all hoses have been traced from the lubrication unit to ensure grease circuits are properly connected. |  |  |  |  |
|  |  | Confirm installation is in accordance with the manufacturer’s instructions. |  |  |  |  |
|  |  | Confirm complete circuit has been checked against the P&ID, schematic drawing & is correct. |  |  |  |  |
|  |  | Confirm grease unit hold down bolts are tensioned correctly. |  |  |  |  |
|  |  | Confirm all pre- assembled bolted connections, including couplings, checked for correct tension after transport. |  |  |  |  |
|  |  | Confirm all flexible hoses not stressed or twisted during installation & have unrestricted movement. |  |  |  |  |
|  |  | Confirm all flexible hoses adequately protected at any possible chafing point. |  |  |  |  |
|  |  | Confirm all hoses have a flexible sheath installed over them to prevent damage or injury in the event of a leak. |  |  |  |  |
|  |  | Confirm all hard piping is adequately supported. |  |  |  |  |
|  |  | Confirm tank cleaned. |  |  |  |  |
|  |  | Confirm all connections tightened. |  |  |  |  |
|  |  | Confirm tank filled as per manufacturer’s specification. Type : __________________ Qty : ___________________ |  |  |  |  |
|  |  | Confirm cabinet located to allow specified clearance on all sides. |  |  |  |  |
|  |  | Confirm grease pump pre filled with grease and all lines purged. |  |  |  |  |
|  |  | Confirm pressure relief valves installed and within test date and correct setting as per P&ID. |  |  |  |  |
|  |  | Confirm all guarding installed. |  |  |  |  |
|  |  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |  |
|  |  | Confirm all grease points on the machine have been greased. |  |  |  |  |
| HYDRAULIC POWER PACK & COOLING UNIT |  |  |  |  |  |  |
|  |  | Confirm all transport protection removed. |  |  |  |  |
|  |  | Confirm thorough inspection undertaken to identify any transport damage. |  |  |  |  |
|  |  | Confirm unit checked for any loose connections after transport. |  |  |  |  |
|  |  | Confirm all hoses have been traced from power unit to the tooth removal head to ensure hydraulic circuits are properly connected. (marked up redlined P&ID attached) |  |  |  |  |
|  |  | Confirm installation is in accordance with the manufacturer’s instructions. |  |  |  |  |
|  |  | Confirm complete circuit has been checked against the schematic drawing & is correct. |  |  |  |  |
|  |  | Confirm hydraulic unit hold down bolts are tensioned correctly. |  |  |  |  |
|  |  | Confirm all pre- assembled bolted connections, including couplings, checked for correct tension after transport. |  |  |  |  |
|  |  | Confirm filter elements installed, flow direction correct & inspected before first fill. |  |  |  |  |
|  |  | Confirm filters installed with correct micron rating. |  |  |  |  |
|  |  | Confirm reservoir breather installed. |  |  |  |  |
|  |  | Confirm all flexible hoses not stressed or twisted during installation & have unrestricted movement. |  |  |  |  |
|  |  | Confirm all flexible hoses adequately protected at any possible chafing point. |  |  |  |  |
|  |  | Confirm all hoses have a flexible sheath installed over them to prevent damage or injury in the event of a leak. |  |  |  |  |
|  |  | Confirm all hard piping is adequately supported. |  |  |  |  |
|  |  | Confirm tank cleaned and flushed and full system flush has been conducted, results attached & looped lines re- connected. |  |  |  |  |
|  |  | Confirm all connections tightened. |  |  |  |  |
|  |  | Confirm tank filled as per manufacturer’s specification. |  |  |  |  |
|  |  | Confirm cabinet located to allow specified clearance on all sides. |  |  |  |  |
|  |  | Confirm return lines connected to tank with minimum restrictions. |  |  |  |  |
|  |  | Confirm lubrication pump pre filled with lubrication oil. |  |  |  |  |
|  |  | Confirm representative oil sample taken and Oil Analysis Report attached. |  |  |  |  |
|  |  | Confirm all pressure control valves are correctly pre- set: Record pressure: ______________ |  |  |  |  |
|  |  | Confirm accumulators inspected & record pre charge pressure. (accumulators to be charged in stage 2) |  |  |  |  |
|  |  | Confirm pressure relief valves installed and within test date and correct setting as per P&ID. |  |  |  |  |
|  |  | Confirm reservoir breather installed. |  |  |  |  |
|  |  | Confirm all guarding installed. |  |  |  |  |
|  |  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |  |
| COOLING UNIT |  |  |  |  |  |  |
|  | Confirm unit installed with the minimum clearances per manufacturer’s recommendations. |  |  |  |  |  |
|  | Confirm all fan guards & access doors in place. |  |  |  |  |  |
|  | Confirm any material or debris removed from top/surrounds of fans. |  |  |  |  |  |
|  | Confirm all transport protection removed. |  |  |  |  |  |
|  | Confirm unit installed level & hold down bolts tensioned correctly. |  |  |  |  |  |
|  | Confirm fans can be rotated freely by hand and confirm not catching with structure/ guards. |  |  |  |  |  |
|  | Confirm all pipe supports installed & clamps/ U-bolts tightened. |  |  |  |  |  |
|  | Confirm all piping installed as per design & specification. |  |  |  |  |  |
|  | Confirm system piping pre flushed before final connection to unit to remove all foreign material & dirt. |  |  |  |  |  |
|  | Confirm all guarding installed. |  |  |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |