| Tag No. «Name» - «Description» |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subsystem: «ProcessBreakdownIdentifier» - «ProcessBreakdownName» |  |  |  |  |
| Manufacturer. | «ManufacturerName» | Model No. |  | «OEMPartNumber» |
| Field to record Serial No. | «SerialNumber» |  |  |  |
| Drawing No. | «DocumentList» | Task No. | «TaskName» |  |
| Additional Information: | «Comments» |  |  |  |

| Item | Description |  |  |  |  |  |  |  |  | OK |  | N/A | Punch |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCREEN |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Inspect all components and confirm they are free from damage. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm screen has been installed in accordance with the design drawings & manufacturer’s instructions and survey attached. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all screen deck protection panels removed and all panels seated properly. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm deck retention bolts checked for tension after transit. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm deck locking strips seated correctly & deflectors point towards feed end. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all spring centre measurements checked to drawing specifications. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm measured length between spring base plate centres within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm measured width between spring base plate centres within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm Spring base plate centres are square – diagonals within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm heights of spring base plates are within ±3 mm. (survey report attached) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all spring base plate bolts have been tensioned in accordance with the manufacturer’s instructions. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all bolts installed with self-locking nuts. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm spring verticality (sag) has been measured & is no greater than 5 mm for ALL springs (use level placed on side of springs). |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm Static Inclination Of Deck: Horizontal (within 4 mm). |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm Static Inclination Of Deck: Longitudinal (as nominated on drawing): - Horizontal (within 4 mm) mm - Longitudinal (as nominated on drawing) mm |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all beam protection covers are installed and not damaged. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm screen media panel apertures are installed correctly as per deck assembly drawings. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all dust covers and seals installed correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all signage & equipment labelling correct & complete as per design drawings and project specifications. |  |  |  |  |  |  |  |  |  |  |  |  |
| SPRING HEIGHTS (measure and record spring length at centre of spring pack, length must be equal within 4 mm side to side) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Feeder | Isolation Frame |  | DISCHARGE FEED |  |  | Feeder |  | Isolation Frame |  |  |  |
| LHD |  | mm | mm |  |  |  | RHD | mm |  | mm |  |  |  |
| LHF |  | mm | mm |  |  |  | RHF | mm |  | mm |  |  |  |
| Item | Description |  |  |  |  |  |  |  |  | OK |  | N/A | Punch |
| ISOLATION FRAME |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm each side frame is level within ±2 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm assembly is square in Plan View – diagonals within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm assembly is square in End View – diagonals within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm measured length between spring base plate centres within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm measured width between spring base plate centres within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm spring base plate centres are square – diagonals within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm heights of spring base plates are within ±3 mm. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm spring verticality (sag) has been measured & is no greater than 5 mm for ALL springs (use level placed on side of springs). |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all bolts have been tensioned in accordance with manufacturer’s instructions. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm all bolts installed with self-locking nuts. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm dampers installed & bolts tensioned. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Confirm screen installed on isolation frame and survey attached. |  |  |  |  |  |  |  |  |  |  |  |  |
| CLEARANCES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clearances on Structure- general as per design and OEM drawings/specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clearance Description |  |  |  | Corner Location |  | Vertical (mm) |  |  | Longitudinal (mm) |  | Transverse (mm) |  |  |
| Isolation Frame Clearance Between Structures (Front is discharge of screen) (LH & RH are respective to the material flow direction) |  |  |  | Rear LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rear RH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front RH |  |  |  |  |  |  |  |  |  |
| Clearance Between Screen and Discharge Chute |  |  |  | Rear LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rear RH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front RH |  |  |  |  |  |  |  |  |  |
| Clearance Between Screen and Feeder |  |  |  | Rear LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rear RH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front RH |  |  |  |  |  |  |  |  |  |
| Clearance Between Screen and Dust Cover Fixed Structures |  |  |  | Rear LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Rear RH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front LH |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Front RH |  |  |  |  |  |  |  |  |  |

Typical Example of Screen Clearances

| EXCITER MODEL No. |  | SERIAL No. DE |  | SERIAL No. NDE |  |
| --- | --- | --- | --- | --- | --- |

| Item | Description | OK | N/A | Punch |
| --- | --- | --- | --- | --- |
| EXCITERS & DRIVE UNIT |  |  |  |  |
|  | Confirm motor hold down bolts tensioned correctly and no soft foot evident. |  |  |  |
|  | Confirm exciter transport protection removed. |  |  |  |
|  | Confirm exciter bolts with nut and washer outside beam, so nut fits into the machined spot face. |  |  |  |
|  | Confirm exciters aligned, phase and weight configuration has been set correctly for design amplitude/throw and all plugs are affixed. |  |  |  |
|  | Confirm exciter hold down bolts tensioned correctly- required to check tension after transport: Torque (Nm) |  |  |  |
|  | Confirm breathers correctly installed and protective seals removed. |  |  |  |
|  | Confirm condition monitoring equipment installed as per design requirements. |  |  |  |
|  | Confirm layshaft bearing hold down bolts tensioned correctly. |  |  |  |
|  | Confirm Horizontal Axis: lay shaft axis is BELOW exciter shaft axis as per nominated measurement on vendor drawing (±5 mm). |  |  |  |
|  | Confirm Vertical Axis: lay shaft axis is in alignment with exciter shaft axis as per nominated measurement on vendor drawing (±5 mm). |  |  |  |
|  | Confirm drive shaft couplings secured & aligned and torqued correctly _________Nm. Attach alignment report. |  |  |  |
|  | Confirm all first fills have been completed, samples taken and results attached. |  |  |  |
|  | Confirm direction of rotation physically checked for both drive & gearbox and marked. |  |  |  |
|  | Confirm all guarding installed with use of self-locking nuts or the use of thread locker where tapped holes are used. (if required by vendor) |  |  |  |
|  | Confirm all identification and signage is installed as per design drawings and project specifications. |  |  |  |
| ROLL OUT UNITS AND RAIL |  |  |  |  |
|  | Confirm all rails are aligned and installed as per specifications and drawings and survey is attached. |  |  |  |
|  | Confirm all roll out drives are affixed correctly and aligned. |  |  |  |
|  | Confirm all wheels rotate freely by hand with no signs of binding. |  |  |  |
|  | Confirm direction of rotation is marked clearly on each drive. |  |  |  |
|  | Confirm each hydraulic ram is free from damage, inlet and outlet fittings are free from debris and sufficiently plugged to stop dirt ingression. |  |  |  |
|  | Confirm all cylinder rods and pins are installed correctly. |  |  |  |
|  | Confirm vendor ITR (M1.099) has been completed for all sections of the installation. |  |  |  |

| COMMENTS, OBSERVATIONS & ATTACHMENTS (Complete a Punchlist Form and attach if any of the checks above marked as “Punch”) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| COMPLETED |  |  |  |  |  |
| I hereby confirm the work has been completed in accordance with Contract specified drawings, specifications & standards. |  |  |  |  |  |
| PERFORMED BY CONTRACTOR: |  | REVIEW BY SUPERVISOR/LEAD ENGINEER: |  | EPCM ACCEPTED BY: |  |
| Signature: |  | Signature: |  | Signature: |  |
| Name: |  | Name: |  | Name: |  |
| Title: |  | Title: |  | Title: |  |
| Date: |  | Date: |  | Date: |  |