| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Check installation against drawings | ☐ | ☐ | ☐ |  |  |  |
| 2 | Thorough visual inspection undertaken for any transport damage including lubrication components | ☐ | ☐ | ☐ |  |  |  |
| 3 | All bearing transport protection removed | ☐ | ☐ | ☐ |  |  |  |
| 4 | All remaining bolted connections checked for correct tension | ☐ | ☐ | ☐ |  |  |  |
| 5 | Check all hoses & cables are secured clear from rotating elements | ☐ | ☐ | ☐ |  |  |  |
| 6 | Check clearance between hopper liner & skirt as per vendor IOM | ☐ | ☐ | ☐ |  |  |  |
| 7 | Clearance between rip detector and belt is 25mm | ☐ | ☐ | ☐ |  |  |  |
| 8 | Distance between belt drift switch and belt is 75mm | ☐ | ☐ | ☐ |  |  |  |
| 9 | Surge bar clearance set to 150mm above belt feeder | ☐ | ☐ | ☐ |  |  |  |
| 10 | Launder sprays activate when feeder is run backwards | ☐ | ☐ | ☐ |  |  |  |
| 11 | Check all rotating elements are free from obstruction | ☐ | ☐ | ☐ |  |  |  |
| 12 | All lubrication lines have been connected & lube tank filled with grease | ☐ | ☐ | ☐ | Grease Type: ____________ Grease Qty: ____________ |  |  |
| 13 | Check grease pump gearbox oil level | ☐ | ☐ | ☐ |  |  |  |
| 14 | All piping connected to grease unit has the correct pressure rating | ☐ | ☐ | ☐ |  |  |  |
| 15 | Feeder locking pins can be engaged & disengaged by use of hand pump | ☐ | ☐ | ☐ |  |  |  |
| 16 | Safety lanyards installed | ☐ | ☐ | ☐ |  |  |  |
| 17 | All guarding in place & secure | ☐ | ☐ | ☐ |  |  |  |
| 18 | All signage in place as per Client Specification | ☐ | ☐ | ☐ |  |  |  |
| 19 | A copy of Vendor or QU-FIC-0006 belt splice check sheet attached | ☐ | ☐ | ☐ |  |  |  |
| 20 | Belt Plough QU-FIC-0026 & Belt Scraper QU-FIC-0018 check sheets have been completed | ☐ | ☐ | ☐ |  |  |  |
| 21 | Hydraulic Brakes QU-FIC-0005 has been completed | ☐ | ☐ | ☐ |  |  |  |
| 22 | Drives QU-FIC-0003 check sheet has been completed | ☐ | ☐ | ☐ |  |  |  |
| 23 | Metal Detector QU-FIC-0030 check sheets have been completed (if applicable) | ☐ | ☐ | ☐ |  |  |  |
| 24 | Pulley check sheet QU-FIC-0021 completed & attached | ☐ | ☐ | ☐ |  |  |  |
| 25 | Idler QU-FIC-0020 check sheet complete | ☐ | ☐ | ☐ |  |  |  |
| 26 | QU-FIC-0002 High Speed and Low Speed Coupling check sheets complete | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |