| - FEEDER SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| - Screen Model No |  | - Vibration Unit Model No |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | GENERAL |  |  |  |  |  |  |
| 1 | Check for any mechanical damage | ☐ | ☐ | ☐ |  |  |  |
| 2 | Screen installed in accordance with the drawings and manufacturers instructions | ☐ | ☐ | ☐ |  |  |  |
| 3 | Material, apertures & number of screen decks installed as per specification | ☐ | ☐ | ☐ |  |  |  |
| 4 | Correct sandwich plates installed | ☐ | ☐ | ☐ |  |  |  |
| 5 | All bolts on mounting springs & stabilizers snug tight | ☐ | ☐ | ☐ |  |  |  |
| 6 | Tuning plate bolts tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 7 | Screen discharge points in alignment with chutes | ☐ | ☐ | ☐ |  |  |  |
| 8 | Screen discharge access door does not clash with discharge chutes when opened | ☐ | ☐ | ☐ |  |  |  |
| 9 | All screen decks, liners & clamp plates secure | ☐ | ☐ | ☐ |  |  |  |
| 10 | General Structure Clearance within manufacturer / client specification | ☐ | ☐ | ☐ |  |  |  |
| 11 | Confirm motor conduit box is fully stuffed with packing to prevent leads from ‘flexing’ as per manufacturer specs | ☐ | ☐ | ☐ |  |  |  |
| 12 | All bolts installed with self-locking nuts | ☐ | ☐ | ☐ |  |  |  |
| 13 | All dust seals installed | ☐ | ☐ | ☐ |  |  |  |
| 14 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |
|  | ELECTRIC VIBRATION UNIT |  |  |  |  |  |  |
| 15 | Unit installed square to screen frame | ☐ | ☐ | ☐ |  |  |  |
| 16 | Mounting bolts tensioned correctly. Bolt torque FIC completed | ☐ | ☐ | ☐ | Bolt Torque: ___________ Nm |  |  |
| 17 | Exciters aligned in phase, survey attached | ☐ | ☐ | ☐ |  |  |  |
| 18 | Exciter cover bolts tensioned correctly | ☐ | ☐ | ☐ |  |  |  |
| 19 | Exciter bearings greased | ☐ | ☐ | ☐ | Brand: __________________ Type: __________________ |  |  |
|  | ISOLATION SPRINGS |  |  |  |  |  |  |
| 20 | Top of all base plates are within manufacturer specification |  |  |  |  |  |  |
| 21 | All spring centre distances checked |  |  |  |  |  |  |
| 22 | Spring centre’s are square – diagonals within manufacturer specification |  |  |  |  |  |  |
| 23 | All base plate bolts tensioned correctly |  |  |  |  |  |  |

| SPRING HEIGHTS (measure and record spring length at centre of spring pack, length must be equal within 4mm side to side) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Flow |  |  |  |  |  |  |
|  | Feeder | Isolation Frame |  |  | Feeder | Isolation Frame |
| LHD | mm | mm |  | RHD | mm | mm |
| LHF | mm | mm |  | RHF | mm | mm |

| CLEARANCES |  |  |  |  |
| --- | --- | --- | --- | --- |
| Feed Chute Clearance | Side L/H: | mm | Side R/H: | mm |
|  | Back L/H: | mm | Back R/H: | mm |
|  | Deck L/H: | mm | Deck R/H: | mm |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |