| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | Model |  | Serial Number |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Check feeders for any transport damage | ☐ | ☐ | ☐ |  |  |  |
| 2 | Feeders are correctly installed in accordance with the drawings | ☐ | ☐ | ☐ |  |  |  |
| 3 | All transport protection removed | ☐ | ☐ | ☐ |  |  |  |
| 4 | Clearances between feeder and all surrounding structure is per specification | ☐ | ☐ | ☐ |  |  |  |
| 5 | Safety beams and slings are correctly fitted – nominal clearance is per specification | ☐ | ☐ | ☐ |  |  |  |
| 6 | Internal pan down slope is set correctly is per specification | ☐ | ☐ | ☐ |  |  |  |
| 7 | Trough is level laterally | ☐ | ☐ | ☐ |  |  |  |
| 8 | All spring hangar bolts fitted with nylock nuts | ☐ | ☐ | ☐ |  |  |  |
| 9 | All liners installed with lock nuts | ☐ | ☐ | ☐ |  |  |  |
| 10 | All dust seals fitted between pan & feed chute | ☐ | ☐ | ☐ |  |  |  |
| 11 | Feeder discharge dust curtain profiled & fitted | ☐ | ☐ | ☐ |  |  |  |
| 12 | Shackle pins on safety slings have been safety wired (moused) using a min. of 1.6mm diameter wire | ☐ | ☐ | ☐ |  |  |  |
| 13 | Rubber seals around feeder installed is per specification | ☐ | ☐ | ☐ |  |  |  |
| 14 | Check tension of electromagnetic vibration unit bolts. Bolt Torque FIC Completed | ☐ | ☐ | ☐ |  |  |  |
| 15 | All signage installed as per client specification | ☐ | ☐ | ☐ |  |  |  |

| SUSPENSION SPRINGS – As viewed from rear of feeder |  |
| --- | --- |
| Front Left | mm |
| Rear left | mm |
| Front Right | mm |
| Rear Right | mm |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |