- If units are vendor supplied fully coupled & aligned, then mark items as “Vendor” & attach appropriate Vendor Factory Test Sheets.

| SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| COUPLING |  | TEST EQUIPMENT |  |
| Manufacturer: |  | Type: |  |
| Model: |  | Asset / Serial No: |  |
| Serial Number: |  | Calibration: |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
| 1 | Is coupling field-fitted | ☐ | ☐ | ☐ |  |  |  |
| 2 | Rotalign or coupling laser alignment to be used; Attach alignment report including equipment calibration certificate | ☐ | ☐ | ☐ |  |  |  |
| 3 | Piping and installation checked for strain | ☐ | ☐ | ☐ |  |  |  |
| 4 | All components free from damage | ☐ | ☐ | ☐ |  |  |  |
| 5 | Coupling assembled in accordance with manufacturers specification / IOM | ☐ | ☐ | ☐ |  |  |  |
| 6 | Coupling free from all foreign material | ☐ | ☐ | ☐ |  |  |  |
| 7 | Flange hubs secured to shaft as per manufacturers instruction | ☐ | ☐ | ☐ |  |  |  |
| 8 | Flange faces mate with no gaps- checked with feeler gauge | ☐ | ☐ | ☐ |  |  |  |
| 9 | Bolts tensioned in accordance with manufacturers recommendation & torque check sheet attached | ☐ | ☐ | ☐ |  |  |  |
| 10 | Adequate guarding securely fitted and clear of all rotating elements | ☐ | ☐ | ☐ |  |  |  |

RADIAL ALIGNMENT

mm mm mm N/S/E/W mm N/S/E/W

ANGULAR ALIGNMENT

mm mm mm N/S/E/W mm N/S/E/W

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |