| SPECIFICATIONS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturer |  | Model |  | Serial Number |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | - WEIGH FRAME |  |  |  |  |  |  |
| 1 | - Belt weigher installed as per vendor specifications & client specification | ☐ | ☐ | ☐ |  |  |  |
| 2 | - Weigh frame installed on horizontal or straight inclined portion of the conveyor | ☐ | ☐ | ☐ |  |  |  |
| 3 | - Minimum clearance achieved between lowest part of weigh frame & return belt | ☐ | ☐ | ☐ |  |  |  |
| 4 | - Weigh area is at a distance from training idlers, water sprays, belt splice stations, conveyor skirting & cross belt samplers, as per vendors specifications | ☐ | ☐ | ☐ |  |  |  |
| 5 | - Weigh frame centreline is located exactly on the conveyor centreline | ☐ | ☐ | ☐ |  |  |  |
| 6 | - Weigh frame cross members level in lateral plane & square to each other | ☐ | ☐ | ☐ |  |  |  |
| 7 | - Load cells located & fixed in position as per vendors specification | ☐ | ☐ | ☐ |  |  |  |
| 8 | - Alignment carried out – Attach vendor report | ☐ | ☐ | ☐ |  |  |  |
| 9 | - Calibration weight set up is clear of the belt | ☐ | ☐ | ☐ |  |  |  |
| 10 | - Calibration weights can be easily engaged & disengaged | ☐ | ☐ | ☐ |  |  |  |
| 11 | - All cabling is run through conduit & is clear from both belt & idlers | ☐ | ☐ | ☐ |  |  |  |
|  | - IDLERS |  |  |  |  |  |  |
| 12 | - Weigh quality idlers used in weigh frame & at least 2 sets of fixed lead in/ lead out idler frames | ☐ | ☐ | ☐ |  |  |  |
| 13 | - Concentricity of weigh quality idlers within ± 0.1mm | ☐ | ☐ | ☐ |  |  |  |
| 14 | - Idler spacings for both lead in & lead out fixed idler sets are as per vendor specification | ☐ | ☐ | ☐ |  |  |  |
| 15 | - Lead in & lead out out fixed idler sets installed square to stringers | ☐ | ☐ | ☐ |  |  |  |
| 16 | - Both lead in & lead out out fixed idlers level in lateral plane |  |  |  |  |  |  |
| 17 | - Weigh frame idler frames have a minimum of 100mm clearance over stringers |  |  |  |  |  |  |
| 18 | - Weigh frame idlers & inner lead in & lead out fixed roller sets set at the correct tolerance from the datum point as per vendor specification |  |  |  |  |  |  |
| 19 | - Standard idlers shimmed to ramp down from datum points within the correct tolerance as per vendor specification |  |  |  |  |  |  |
| 20 | - All Idler frame bolts tensioned correctly |  |  |  |  |  |  |
|  | - TACHO / FRICTION WHEEL |  |  |  |  |  |  |
| 21 | - Tacho/ friction wheel positioning is as per vendor specification |  |  |  |  |  |  |
| 22 | - Tacho/ friction wheel has sufficient tension to underside of carry belt/ tail pulley |  |  |  |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |