| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | GENERAL |  |  |  |  |  |  |
| 1 | Check installation against drawings | ☐ | ☐ | ☐ |  |  |  |
| 2 | Thorough visual inspection undertaken for any transport damage including lubrication components | ☐ | ☐ | ☐ |  |  |  |
| 3 | All bearing transport protection removed | ☐ | ☐ | ☐ |  |  |  |
| 4 | All remaining bolted connections checked for correct tension QU-FIC-0054 & QU-FIC-0075 attached as applicable | ☐ | ☐ | ☐ |  |  |  |
| 5 | Check all hoses & cables are secured clear from rotating elements | ☐ | ☐ | ☐ |  |  |  |
| 6 | Check clearance between hopper liner & skirt as per vendor IOM | ☐ | ☐ | ☐ |  |  |  |
| 7 | Check all rotating elements are free from obstruction | ☐ | ☐ | ☐ |  |  |  |
| 8 | All lubrication lines have been connected & lube tank filled with grease | ☐ | ☐ | ☐ |  |  |  |
| 9 | Check grease pump gearbox oil level | ☐ | ☐ | ☐ |  |  |  |
| 10 | All piping connected to grease unit has the correct pressure rating | ☐ | ☐ | ☐ |  |  |  |
| 11 | Feeder locking pins can be engaged & disengaged by use of hand pump | ☐ | ☐ | ☐ |  |  |  |
| 12 | Safety lanyards installed | ☐ | ☐ | ☐ |  |  |  |
| 13 | All guarding in place & secure | ☐ | ☐ | ☐ |  |  |  |
| 14 | All signage in place as per Client Specification | ☐ | ☐ | ☐ |  |  |  |
| 15 | A copy of Vendor or QU-FIC-0006 belt splice check sheet attached | ☐ | ☐ | ☐ |  |  |  |
| 16 | Pulley check sheet QU-FIC-0021 completed & attached | ☐ | ☐ | ☐ |  |  |  |
| 17 | Hydraulic Motor & Power Pack installed & check sheet FIC-0052 attached | ☐ | ☐ | ☐ |  |  |  |
| 18 | Belt Plough QU-FIC-0026 & Belt Scraper QU-FIC-0018 check sheets have been completed | ☐ | ☐ | ☐ |  |  |  |
| 19 | Metal Detector QU-FIC-0030 check sheets have been completed (if applicable) | ☐ | ☐ | ☐ |  |  |  |

| INSPECTION ITEMS |  | YES | NO | N/A | REMARKS | SIGN OFF |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | CIVMEC | CLIENT |
|  | IDLERS |  |  |  |  |  |  |
| 20 | Check idler bolts. If any found loose, idler alignment check to be undertaken | ☐ | ☐ | ☐ |  |  |  |
| 21 | Ensure all breather holes are at the lower end for all inclined (wing) rollers | ☐ | ☐ | ☐ |  |  |  |
| 22 | All training idler frames and guide rollers checked for correct orientation | ☐ | ☐ | ☐ |  |  |  |
| 23 | Non-magnetic idlers used in proximity of metal detector | ☐ | ☐ | ☐ |  |  |  |
| 24 | Check all rollers are seated to a full depth of the slot in the support frame | ☐ | ☐ | ☐ |  |  |  |
| 25 | Check for any oil leaks from idler seals | ☐ | ☐ | ☐ |  |  |  |
|  | RETRACTION SHUTTLE (IF APPLICABLE) |  |  |  |  |  |  |
| 26 | Shuttle rail alignment/ survey attached | ☐ | ☐ | ☐ |  |  |  |
| 27 | Retraction drive chain/ track bolts checked for correct tension | ☐ | ☐ | ☐ |  |  |  |
| 28 | Retraction shuttle drive shaft bearings have been greased | ☐ | ☐ | ☐ |  |  |  |
| 29 | Drive sprockets aligned & engages track | ☐ | ☐ | ☐ |  |  |  |
| 30 | Travel rails installed level & aligned | ☐ | ☐ | ☐ |  |  |  |
| 31 | Travel wheels are aligned with track | ☐ | ☐ | ☐ |  |  |  |
| 32 | Feeder is seated on all support wheels | ☐ | ☐ | ☐ |  |  |  |
| 33 | Hydraulic Brakes QU-FIC-0005 has been completed | ☐ | ☐ | ☐ |  |  |  |

COMMENTS:

| CIVMEC | Name | Signed | Date |
| --- | --- | --- | --- |
| CLIENT | Name | Signed | Date |